# SecPriPC Zusammenfassung

Made by El Presidente

Keine Garantie auf Falschheit oder Korrektheit.

Das ist meine allerletzte Zusammenfassung, nutze sie weise! (oder auch nicht).

!!!

Für alle die sagen dass der Kram hier zu lang ist: Schaut ans Ende, dort sind Klausurfragen aus der Probeklausur und den Folien inklusive Antworten drinnen.

!!!

1. Introduction	7

Pervasive Computing	7

4 Enablers of Pervasive Computing	7

IoT (Internet of Things)	7

Wireless Technology	7

Definition: Security	7

Security Goals (CIA) and Authentication and Authorization	8

Evaluating Attackers	8

Privacy	8

Security & Privacy in IoT	8

5 Assessments of S&P	8

5 Design principles for pervasive systems (by Adam Greenfield)	9

Need to know:	9

2. Cellular: GSM	10

Privacy in Mobile Networks	10

GSM and its 4 features	10

GSM cellular networks	10

GSM Architecture	10

Handover Decision	12

Call Setup	12

GSM S&P Assessment	12

GSM Threats and Attackers	13

4 GSM Security Features	13

GSM Security Risks	13

GSM Authentication and Voice/SMS Encryption	13

Does GSM Security Prevent Threats?	15

Free call attack: Ross Anderson’s Hack	15

Free call attack: SIM Card Cloning	15

A3/A8 Security by Obscurity	16

A3/A8/COMP128 Replacement	16

Kerckhoffs’ Six Principles	16

Security by Obscurity	16

AES (Advanced Encryption Standard)	17

Attack: Stolen or Lost MS	17

Attack: Backend Eavesdropping	17

How to Eavesdrop? => IMSI Catcher	17

GSM: Lessons Learned	18

Need to know	19

3. Cellular: UMTS Architecture and Security	21

UMTS (Universal Mobile Telecommunications System)	21

UMTS Architecture	21

UMTS Authentication and Key Agreement (14 steps)	22

Lessons Learned	24

Need to know	24

4. Cellular: SS7, LTE, Location privacy	25

SS7 Protocol	25

Attack: Rogue SS7 Operator	25

Attack: Stealing Money via SS7	25

Location Privacy	25

Location Triangle: Who, Where, When	25

Data Retention EU (Vorratsdatenspeicherung)	26

GSM / UMTS Tracking Possibilities	26

LTE/4G	26

LTE Architecture	27

LTE Connection Setup	27

Attack: Impersonation (blling fraud) in misconfigured LTE	27

LTE Privacy: 3 Tradeoffs	28

LTE Availability: 1 Tradeoff	28

Cryptography Basics	28

LTE Integrity	30

Attack: aLTEr Stream Cipher attack	30

Need to know:	31

5. Cellular: Attacks on LTE and 5G	32

Attack: Eavesdropping Encrypted LTE Calls (ReVoLTE)	32

5G Security Issues (in a nutshell)	32

5G Authentification	32

5G AKA Protocol	33

Cellular Security & Privacy: 12 Lessons Learned	34

Need to know:	35

5. Wi-Fi: Wi-Fi, OAN, WEP	36

Wi-Fi (Wireless Fidelity, IEEE 802.11 since 1997)	36

Network Types: Infrastructure and Ad-hoc	36

Wi-Fi Network Infrastructure	37

Connecting to Wi-Fi Network	37

Attacks: Open Network Protection -How to prevent alien STAs from joining the WLAN - and how an attacker can join anyway	39

Attack: Rogue Access Point (Evil Twin Attack)	39

WEP (Wired Equivalent Privacy, 1999)	40

Cryptography Basics: Stream Ciphers	40

Cryptography Basics: Pseudo Random Number Generator (PRNG)	41

WEP Encryption and Decryption	41

Attack on WEP: - Cryptographic Attacks	42

Attack on WEP: Non-crypto Attacks	42

Need to know:	42

6. More Wi-Fi: WEP	43

One Time Pad	43

Cracking Security if Two Times Pad is used	43

WEP: Initialization Vector (IV)	43

Attack on WEP: IV Reuse	44

Attacks on WEP: Two other IV-based Vulnerabilities	44

Attack on WEP: CRC-based Attack	44

Attack on WEP: Replay Attack on Access Control	45

Summary: 4 WEP Design Issues	45

WEP: 4 Lessons Learned	46

Need to know:	47

6. More Wi-Fi: WPA and WPA2	48

WPA and WPA 2 (Wireless Protected Access)	48

WPA2 Key Hierarchy	48

Four-Way Handshake	49

Best practices for Authentication and key management protocols	49

Attack: WPA2-PSK Key Cracking	50

Attack: Insider Attacks on WPA2-PSK	50

Attack: Insider Attack of WPA2 via Hole 196	51

Attack: Key Reinstallation Attack	52

WPA2 Security Summary	53

Need to know:	53

7. More Wi-Fi	54

WPA3	54

Attack on WPA3: Dragonblood Attacks	54

Defenses against Downgrading	55

Lessons Learned: WPA2/3	55

Need to know:	55

7. Zigbee	56

IEEE 802.15.4	56

ZigBee Versions/Timeline	56

Clusters & Profiles	56

ZigBee Home Automation (ZHA) Profile	56

ZigBee Light Link (ZLL) Profile	56

ZigBee PANs (simplified)	56

ZigBee Nodes & Networks	57

Security Goals of ZigBee	57

Attacker Model	57

Keys in ZigBee 3.0	58

Network Types and Link Keys	58

Touchlink (ZLL) Master Key	58

EZ-Mode Commissioning	58

Touchlink Commissioning	59

Network Key Encryption	60

Inter-PAN frames	60

Active Device Scan	60

Attack: Identify Action Attack	60

Need to know:	61

8. More Zigbee	62

Attack: Reset to Factory-New Attack	62

Attack: Permanent Disconnect Attack	62

Attack: Hijacking - Attack with knowledge of the leaked global master key	63

Attack: Network Key Extraction - Attack with knowledge of the leaked global master key	63

ZigBee Proximity Check	63

Bug in Proximity Check in Smart Bulbs	63

Attack: Using Proximity Chek Bug to factory-reset Devices	63

Conclusion of ZigBee Security Analysis	63

Lessons Learned ZigBee (3+3)	64

Establishing Economic Incentives for Security Patching of IoT Consumer Products	64

Need to Know:	64

9. Bluetooth and Device Pairing	65

Device Pairing	65

(Original) Bluetooth Idea	65

Bluetooth Piconet (System Architecture)	65

Bluetooth profiles	66

Bluetooth Scatternet (System Architecture)	66

Bluetooth Security vs GSM/UMTS/LTE and Wi-Fi	66

Legacy Bluetooth Security	67

Bluetooth Key Hierarchy	67

Initialization Key Generation and Authentication	67

Link Key Generation	68

Link Key Update	68

Encryption Keys	68

Bluetooth 1.0-2.0 Vulnerabilities	68

Attack on older Bluetooth: Passive PIN Cracking	69

Seven Bluetooth 1.0-2.0 Vulnerabilities	69

Six Bluetooth Lessons Learned (or confirmed)	70

SSP: Secure Simple Pairing (starting with Bluetooth v2.1)	70

Cryptography: Asymmetric Crypto Key Exchange	70

Diffie-Hellman (DH) Key Exchange	71

DH Key Exchange Security	72

Attack: Man-in-the-Middle Attack on DH	72

Authenticated DH (+MANA)	72

Need to know:	73

10. More Bluetooth	74

Bluetooth 2.1+ Pairing: Numeric Comparison	74

Basic Numeric Comparison	74

Bluetooth 2.1+ Pairing: Passkey Entry	75

Basic Passkey Authentication Protocol	76

Bluetooth 2.1+ Pairing: OOB (out-of-band)	77

Bluetooth 2.1+ Pairing: Just works	77

Attack: Degradation to Just Works Attack	77

Attack: Method Confusion Attacks	77

Unauthenticated DH in BLE	78

Bluetooth Pairing: 6 Lessons Learned	78

Need to know:	79

10. Device pairing	80

Device Pairing	80

Possible OOB Channels	80

WPS: WiFi Protected Setup + WPS Options	80

Attack: WPS AP-PIN Cracking	81

Need to know:	81

11: Device pairing in research and OOB channels	82

Resurrecting Duckling	82

Network-in-a-box (NiaB)	82

Seeing Is Believing	82

Shake Well Before Use	83

T2Pair (Touch to Pair)	83

IoTCupid	84

Object-based Pairing: Wanda Idea	84

Lessons Learnt	85

Need to know:	85

12. RFID	87

RFID (Radio-Frequency Identification)	87

Passive RFID	87

Battery Assisted Passive (BAP) RFID	87

Active RFID	87

Passive RFID with Sensors	87

RFID Applications	88

Barcodes	88

Barcodes vs. RFID	88

EPC (Electronic Product Code)	88

EPCglobal	88

EPCglobal Tag Format	89

EPC: Reading Multiple Tags Issues	89

EPC RFID Tag Collision	89

Query Tree Protocol (EPC Generation 1)	89

Query Slot Protocol (EPC Generation 2)	90

Query Tree vs. Query Slot	91

4 Security Threats in EPC	91

RFID Malware	91

7 Privacy Threats in EPC	92

Need to know:	92

13: More RFID	94

Tag Killing (Ensuring Privacy?)	94

Tag Covering (Ensuring Privacy?)	94

Blocker Tag (Ensuring Privacy?)	94

Selective Blocker Tags	94

RFID Bill of Right	95

RFID: Lessons Learned (so far)	95

Electronic Documents with RFID or other proximity cards	95

Passport Access Control Protocols	96

Basic Access Control	97

Example: Cloning E-Passport, 2006	97

Example: Cloning E-Passport, 2007	97

Example: Cloning and Changing E-Passport, 2008	97

Tracking via Replay Attack	98

E Documents - Lessons learned	98

Ghost-and-Leech Attacks on Access Control and Payment Systems	98

Skim Clone RFID Tags of car key	100

Lessons Learned	100

Need to know:	100

Social Lectures	102

Exam Questions/Tasks	103

Introduction	103

Cellular Communication	107

Wi-Fi	112

What are the most important lessons learned about pervasive security and privacy on the example of WEP?	112

Key Exchange and Pairing	119

Bluetooth	122

Zig-Bee	128

RFID	134

Need to know	138

# 1. Introduction

## Pervasive Computing

Information processing (including sensing), networking and response anywhere, anytime

- Pervades everyday life

- Context-awareness

- Sensors deliver information about physical environment

- Actors (actuators) response to physical environment

## 4 Enablers of Pervasive Computing

- Miniaturization of computing & storage

- Mobility through wireless networking

- Computing & storage steadily getting cheaper and faster

- Advanced user interfaces

## IoT (Internet of Things)

= Things of our daily live contain computers and are connected via e.g. the Internet or other networks to exchange data and information. Additionally, sensors of the devices deliver information about the physical environment. Actors (actuators) respond to the physical environment.

## Wireless Technology

WAN = Wide Area Networks: GSM / UMTS / LTE,4G,5G / SS7

WLAN = Wireless Local Area Network: Wi-Fi

WPAN = Wireless Personal Area Network: Bluetooth, ZigBee, RFID

## Definition: Security

Protect the right thing in a right way (Ross Anderson)

- Goals: what to protect

- Threats: against what/whom to protect

- Means: how to protect

## Security Goals (CIA) and Authentication and Authorization

Confidentiality

- Protect data from unauthorized reading access

Integrity

- Protect data from unauthorized changes

Availability

- Make data always available on request by an authorized entity

Authentication

- Distinguish between authorized and unauthorized entities

## Evaluating Attackers

- Actors: who would be interested in attacking?

- Resources: Skills, time, money, technology, manpower

- Incentives: why would they attack?

- Damage: consequences of an attack

## Privacy

First definition: bodily / territorial privacy (The Right to be let alone)

Information Privacy: is the claim of individuals, groups, or institutions to determine for themselves when, how, and to what extent information about them is communicated to others.

## Security & Privacy in IoT

- Data Types:  Location, video, audio,  …. => Profiling/Habits => Territorial Privacy issue, as IoT devices are small and everywhere

- Data/System Accessibility: System ownership, devices are always on, data collection & attack surface

- Interaction with devices/systems: Invisible systems/interactions, availability + integrity more important than confidentiality

## 5 Assessments of S&P

- What is the system? (Assets, Stakeholders, what informations can be extracted from the system)

- S&P Goals (for stakeholders and assets)

- Other goals for stakeholders? (functionality, costs, …)

- What is the attacker model? (see “Evaluating Attackers”)

- What are the S&P trade-offs? (all these goals, likelihood and costs)

## 5 Design principles for pervasive systems (by Adam Greenfield)

- Default to harmlessness: Physical, psychic, financial safety always warranted

- Even in case of system failure!

- Graceful degradation of services

- Failure of a part of a system causes as few service degradation as possible

- Be self-disclosing

- Ownership, usage, capabilities must be easy to find out

- Save users’ face

- Never embarrass (blamieren) or harass (belästigen) the user

- Save users’ time

- Provide high usability

- Be deniable

- Opt-out always possible

## Need to know:

Pervasive Computing (5 parts)

IoT Enablers (4 parts)

IoT (5 parts)

WAN (+3/4 Examples)

WLAN (+ 1 Examples)

WPAN (+ 3 Examples)

Security (Definition)

Security Goals (CIA) + Authentication

4 Parts of evaluating attackers

Privacy (3 definitions)

S&P in IoT (3 parts)

5 steps of S&P Assessment

5 Design Principles of pervasive systems (A. Greenfield)

# 2. Cellular: GSM

## Privacy in Mobile Networks

- Cellular telephony is a ubiquitous system

- Controlling telecom operators is the key to full control over citizens

## GSM and its 4 features

GSM (Global System for Mobile Communications)

- Communication: voice and data services

- Total mobility (international, different providers)

- Worldwide connectivity

- High transmission quality (audio quality and reliability even when using trains/cars)

## GSM cellular networks

segmentation of the area into cells

- Use of several carrier frequencies

- Cell sizes: from some 100 m up to 35 km depending on user density, geography,

- transceiver power etc

- Handover: mobile user changes cell, handover of connection to the neighbor cell

## GSM Architecture

| Subystem: | Structure: |
| --- | --- |
| NSS (network and switching subsystem): Call forwarding, handover, switching |  |
| OSS (operation subsystem) Management of the network |  |
| RSS (radio subsystem) Covers all radio aspects |  |

### Radio Subsystem RSS

| MS (mobile station): IMEI, SIM card |  |
| --- | --- |
| BTS: base transceiver station |  |
| BSC: base station controller – Manages several BTSs |  |

#### IMEI (international mobile equipment identifier)

#### SIM card

- IMSI: international mobile subscriber identifier, 64 bit

- Symmetric cryptographic key for authentication and session key agreement for voice and SMS encryption

### Network and Switching Subsystem NSS + Operation Subsystem OSS

| HLR: home location register – Knows the current MSC/BSC of the subscriber – EIR: equipment identity register contains IMEIs of all registered customers – AuC: authentication center contains symmetric cryptographic keys shared with the SIM cards |  |
| --- | --- |
| MSC: mobile switching center – Manages several BSCs |  |
| VLR: visitor location register (database at MSC Receives data from HLR for subscribers currently connected to its BTSs |  |
| GMSC: gateway MSC |  |

#### BTS = Base Transceiver Station

Often on buildings as antennas

#### BSC = Base Station Controller

#### MSC = Mobile Service Switching Center

both inside buildings or on small radio towers (boxes, medium size)

#### Network Subsystem + Operation Subsystem

## Handover Decision

| • Mobile station (MS) regularly checks the signal strength of all base transceiver stations (BTS) in its range. • If a mobile station (MS) exits cell region, it signals a change of BTS to the mobile service switching center (MSC). • This results in a changed region id in the home file of the user in the home location register (HLR). • Mobile station (MS) always connects to the most powerful base transceiver stations (BTS). |  |
| --- | --- |

## Call Setup

| If somebody calls a mobile station .. | If the mobile station wants to set up a call .. |
| --- | --- |
| – the system looks into the home file of the user – the system identifies the cell region – all base stations in the cell region broadcast a call request to the mobile station – mobile station answers to request – call is set up over base station with the best reception characteristics | – it connects to the current base station and sends the phone number of the called party – the system looks up the current cell region of the called party – the system sets up the call over the corresponding switching stations |

## GSM S&P Assessment

| Stakeholders | Telecom. companies Customers Device Manufacturers State |
| --- | --- |
| System goals | Fast connection establishment, calls to fixed telephone networks, roaming, text messages, online banking, transparent billing system |
| Assets and collected data | Devices and their content, communication metadata (including customer location), communication content, billing, infrastructure |
| Security goals | CIA of all assets + Non-repudiation of calls, privacy of subscribers |

## GSM Threats and Attackers

| Providers | Threats | “free” calls, disruption of communication, |
| --- | --- | --- |
|  | Attackers | customers, criminals and saboteurs, business rivals |
| Customers | Threats | paying for alien or non-existing calls, loss or theft of devices, eavesdropping and tracking |
|  | Attackers | criminals, providers, intelligence agencies, (totalitarian) states |
| Manufacturers | Threats | loss of availability, vandalism |
|  | Attackers | saboteurs, terrorists, business rivals |
| State | Threats | untraceable / anonymous calls, no possibility for wiretapping |
|  | Attackers | criminals, terrorists, other states |

## 4 GSM Security Features

- Subscriber identity (IMSI) authentication

- Subscriber data confidentiality

- Subscriber location privacy

- Signaling information confidentiality (IMSI, IMEI, phone numbers)

Security features can be implemented in BSC and/or MSC

## GSM Security Risks

- misuse of their resources by unauthorized persons using manipulated Mobile Stations, who try to impersonate authorized subscribers; and

- eavesdropping of the various information which are exchanged on the radio path

## GSM Authentication and Voice/SMS Encryption

- Identifiers: Mobile phone (IMEI), SIM card: IMSI, TMSI (temporary mobile subscription identity)

- Symmetric algorithms: In the past mobile phones were not powerful enough for asymmetric public key crypto

- Authentification: A3 and A8 Algorithm, with pre-shared key KSIM

- Encryption: A5, session key KC 64 or 54 bits (key is established for every connection)

### Steps 1-4

IMEI: International Mobile Equipment Identity. It is a unique identification number assigned to every mobile device

IMSI (International Mobile Subscriber Identity): This is a unique identification associated with a SIM card in a mobile device.

TMSI (Temporary Mobile Subscriber Identity): TMSI is a temporary identifier assigned to a mobile device by the network when it connects to the network.

BSC: Base Station Controller

MSC: Mobile Switching Center

HLR: Home Location Register

### Steps 5-8

### Step 9

## Does GSM Security Prevent Threats?

No, free calls, DoS, paying for alien/non-existing calls, eavesdropping and tracking are possible by criminals, providers, state actors.

## Free call attack: Ross Anderson’s Hack

0. Eavesdrop an IMSI from elsewhere (Base Stations can ask for IMSI instead of TMSI)

1. Send IMSI to BSC

2. BSC/MSC send authentication data request to HLR

3. HLR generates SRES via a RAND and a key K

4. HLR sends 5 pairs of RAND, KC and SRES to MSC/BSC

Between MSC and BSC, communication is unencrypted, SRES, RAND and KC can be eavesdropped by attacker

5. BSC sends RAND as a challenge to mobile phone

6. Attackers sends the eavesdropped SRES

## Free call attack: SIM Card Cloning

Option 1: Extract the key KSIM from the smart card…but SIM Cards are smart cards, and smart cars are tamper proof => Secret hard to extract

Option 2: Cryptographic Attack

- Option 2.1: Get physical access to SIM card, break the crypto algorithms A3/A8

  - Submit many RAND queries, SIM card response with SRESs,  analyze SRES response

  - If A3/A8 is cryptographically secure against chosen plaintext attacks, this attack should be infeasible

  - A3/A8 is NOT cryptographically secure

- Option 2.2: Over-the-air cloning (OTA): find out KSIM  from communication

  - Same as above, but with a more restricted number of RAND; SRES pairs

  - Eavesdrop on (RAND, SRES) paris, break the crypto algorithms A3/A8

## A3/A8 Security by Obscurity

The authentication algorithms A3/A8 were kept secret, and they were based on COMP128

Design partially leaked in 1997, the rest was reverse engineered

Attacked using well-known standard techniques

=> A3/A8 was cracked and very insecure because it relied on obscurity, and GSM operators had to replace the algorithm

## A3/A8/COMP128 Replacement

All SIM cards & back end had to be replaced over several months/years, as COMP128 was implemented in hardware.

## Kerckhoffs’ Six Principles

- The system must be practically, if not mathematically, indecipherable;

- It must not be required to be secret, and it must be able to fall into the hands of the enemy without inconvenience; only the cryptographic key(s) should be kept secret

- Its key must be communicable and retainable without the help of written notes, and changeable at the will of the correspondents;

- It must be applicable to telegraphic correspondence;

- t must be portable, and its usage and function must not require the concourse of several people;

- Finally, it is necessary, given the circumstances that command its application, that the system be easy to use, requiring neither mental strain nor the knowledge of a long series of rules to observe.

Principle 2: foundation of modern cryptography

## Security by Obscurity

= keeping secret which algorithm is used for encryption. If the algorithm is known though, the systems can be compromised via cryptographic attacks, as the algorithm might be “bad”/broken.

=> SbO is bad, cryptographic algorithms should instead be subject to open analysis

## AES (Advanced Encryption Standard)

AES algorithm was determined in the AES competition, it uses symmetric cipher with 128-bit key, (cipher = method to encrypt and decrypt data)

## Attack: Stolen or Lost MS

Attack: Attackers steals MS (=steals mobile phone)

Countermeasures:

- Use of authentication for mobile phone and SIM card (PINs, patterns, biometrics)

- IMEI blacklisting (this can also be used to track criminals)

  - Requires to extract and write down the IMEI first

## Attack: Backend Eavesdropping

Eavesdrop on the microwave link between base station and mobile switching center.

This link is was usually left unencrypted, Eavesdrop directly or find out key KC

### Eavesdropping on the A5 Key

### A5 Security by Obscurity

A5/1: Encryption algorithm A5/1 kept secret, secret was leaked, A5/1 could then be cracked with a powerful server.  (key size: 64 bit)

A5/2: Intentionally lower security (key 54 bit). Can be decrypted in real time on a PC, and was prohibited/deprecated in 2007

## How to Eavesdrop? => IMSI Catcher

### Background

Mobile phone can’t eavesdrop other mobile phone, as both transmit on one frequency range (X), but receive on other frequency range (Y)

X transmitter and Y receiver both built directly into mobile phone hardware => no X receiver => no eavesdropping

### IMSI Catcher

IMSI catcher: = portable base station.

- Mobile Phone always connects to BS with strongest signal!

- Base station can require MS to send its real IMSI instead of TMSI

- Tracking using IMSI always possible!

- Sometimes it allows eavesdropping

- Works on GSM, UMTS and LTE

- Used by police and intelligence agencies (fake phone towers in Washington DC)

- GSM network tells MS which encryption algorithms to use => unciphered connection can be enforced (=A5/0), or one that can be cracked (A5/1)

### Eavesdropping with IMSI-Catcher

## GSM: Lessons Learned

- No “security by obscurity” in cryptography => Employ real cryptographers for system design and analysis, and use well-established ciphers

- Provide mutual authentication of MS and GMS network parts to avoid man in the middle attacks

- Provide security in every part of the system (CIA everywhere!)

- Crypto algorithm should be easy to change in a system (in case a cipher breaks one day)

- Consider future tech developments and new more powerful attackers in threat analysis

- Provide transparent technology development processes

## Need to know

GSM (+4 of its features/properties)

GSM cellular network

GSM Architecture:

* NSS with OSS

* HLR

* EIR

* AuC

* MSC

* VLR

* GMSC

* RSS

* MS

* BTS

* BSC

Handover Decision

Call setup

GSM S&P Assesement (4 parts)

GSM Threats, Victims (4 examples) and Attackers

4 GSM Security features

2 GSM Security Risks

9 steps of GSM athentification and voice/SMS encryption

* Grafik!

* IMEI

* IMSI

* TMSI

* BSC

* MSC

* HLR

* Encryption Algorithms

Free Call Attack: Ross Anderson's Hack (7 steps)

Free Call Attack: SIM Card Cloning (2 Options)

A3/A8 Algorithms (secure?)

Kerckhoff's Six Principles (name the mos timportant principle)

Security by Obscurity (+how to do it right)

AES

Attack: Stolen or Lost MS and Countermearues

Attack: Backend Eavesdropping

IMSI Catcher + Countermeasure

# 3. Cellular: UMTS Architecture and Security

## UMTS (Universal Mobile Telecommunications System)

More secure successor of GSM:

- Mutual authentication between MS and network

- Confidentiality and integrity of signaling data

- Confidentiality of user data

- Better algorithms: A5/3 and A5/4

  - Academically broken, but practical attacks in UMTS networks is doubtful

- Backwards compatible to GSM

  - Allows man-in-the-middle attack

## UMTS Architecture

## UMTS Authentication and Key Agreement (14 steps)

### Steps 1-3

### Steps 4-8

### Steps 9-14

### Attack: Man-in-the-middle GSM-UMTS Degredation Attack

• Assumption: MS implements both GSM and UMTS

• IMSI-catcher impersonates MS in UMTS mode to RNC

• RNC sends to MS: “fresh” authentication token AUTN

– IMSI-catcher breaks up the connection, saves AUTN

Now we have a valid AUTN…

Why is this possible?

Integrity protection of the “security algorithms” command is not possible

– GSM does not support integrity protection of signaling messages

## Lessons Learned

(see previous chapter) +

Implementing backward compatibility can leave old vulnerabilities exploitable

## Need to know

UMTS

UMTS Architecture

MS

USIM

NodeB

RNC

MSC

SGSN

VLR

HLR

EIUR

AUC
SS7

UMTS Authentication and Key Agreement (14 steps)

Man-in-the-middle GSM-UMTS Degradation Attack (7 steps)

Why does GSM-UMTS Degradation Attack work?

Problem of backward compatibility

# 4. Cellular: SS7, LTE, Location privacy

## SS7 Protocol

- Developed in 1980 for communication between telecom operators

  - Trust Assumption: everybody is trusted, nos security needed

  - No authentication, no plausibility states

- Current state: used for GSM/UMTS, everybody can buy access for SS7

## Attack: Rogue SS7 Operator

- Locate & track:

  - Ask HLR for IMSI of phone number

  - Ask HLR which MSC is this IMSI

  - Ask MSC: At which BTS is this IMSI

- Eavesdrop: “Please send authentication and encryption keys for this TMSI”

- Manipulate: “This IMSI wants its calls/SMS forwarded to my network”

## Attack: Stealing Money via SS7

- Attacker gains control over victim’s online banking account (via phising or malware)

- Attacker looks up victim’s phone number (Online banking with OTP(one-time-password) via SMS)

- Sets up SMS redirect

- Logs into online banking

- Start translation, use SMS OTP to verify transaction => Money!

## Location Privacy

= the ability to prevent other parties from learning one’s current or past location.

## Location Triangle: Who, Where, When

- Location + Time known: Where have you been at which time

=> Identification of person possible (Nights => At home, Workdays => in the office, …)

- Who + At Which Time => Predict where a person is or might be

- Who + where: predict which time (Max is at the office => it’s a workday between 8 and 16)

- Mobile Profiling and Surveillance possible, and crime investigation (Capitol Attack 2021)

## Data Retention EU (Vorratsdatenspeicherung)

2006: All traffic data (not content!) must be stored for 6 months, and accessible for law enforcement, no additional data may be generated

2007: Transfer of the Data Retention law to national law for telecommunication

2007: Constitutional complaint was filed at the Bundesverfassungsgericht (basic privacy violation, cost in no relation to use)

2009: Transfer for internet

2010: Bundesverfassungsgericht decides that data retention is against German Constitution

2012: EU takes legal action against Germany

2014: EuGH rejects EU-wide data retention law as its being against the EU Charta of human rights

2015: New Data retention law in Germany, next round of complaints

2016: EuGH rejects data retention

2017: data retention stopped in Germany

2022: EuGH conforms rejection

6 Data Things Stored by Operators (telephony, mobile telephony, internet)

- Phone number of caller and callee

- Start and end of connection

- IMSI & IMEI of caller and callee

- Cell id of caller and callee at the beginning of the connection

- IP Address of caller and callee (Internet Telephony)

- Begin and end of access to Internet and IP address of the user

## GSM / UMTS Tracking Possibilities

IMSI Catcher: Pinpoint targeted person with precision up to several meters

TMSI: TMSI can be switched off by BS, TMSI is rarely changed

IMEI: BS can ask mobile phone to transmit their IMEI (feature against phone theft)

## LTE/4G

- LTE = 4G

- MS → UE: User Equipment

- TIMSI → GUTI: Globally Unique Temporary Identifier

- NodeB → eNodeB (e for evolved)

- MSC → MME: Mobility Management Entity

- …

## LTE Architecture

- Mutual authentication, integrity, confidentiality, location privacy

- MME selects crypto algorithm, runs authentication with UE, keeps track of locations

- AES can be used for encryption and integrity protection (other algorithm are also possible)

- Communication between eNodeB and EPC, and inside EPC protected with IPSec

## LTE Connection Setup

## Attack: Impersonation (blling fraud) in misconfigured LTE

## LTE Privacy: 3 Tradeoffs

- GUTI management similar to TMSI management in UMTS/GSM

  - Globally Unique Temporary Identity

  - Rarely changed: remains the same for several days, even when moving

  - Trade-off performance ↔ privacy

- Location leaks via paging requests to eNodeBs

  - Is UE with this GUTI somewhere in the vicinity?”)

  - Unexpected effects of new functionality: smartphone apps

  - Facebook messages (even from non-friends)

  - WhatsApp: just typing (but not sending) a message

- Location leaks by impersonating eNodeBs

  - UE can be asked to send signal strengths of all eNodeBs it can hear without authentication

  - Reason: troubleshooting, but can be used for trilateration

  - Trade-off availability/reliability ↔ privacy

  - Messages can also include GPS coordinates of UEs

## LTE Availability: 1 Tradeoff

- eNodeB impersonation

  - eNodeB messages of the type “service X not allowed” do not require authentication

  - “LTE services not allowed” can be used for

    - Degradation attacks to 2G/3G

    - Permanent DoS (till UE reboot)

- Trade-off availability ↔ security

## Cryptography Basics

Differences between symmetric and asymmetric?

- Symmetric cryptography:

  - Uses a single key for both encryption and decryption.

  - Faster compared to asymmetric cryptography.

  - Typically used for encrypting large amounts of data.

  - Requires secure key exchange between parties.

  - Examples include AES (Advanced Encryption Standard) and DES (Data Encryption Standard).

  - Symmetric: MACs = Message Authentication Codes

- Asymmetric cryptography:

  - Uses a pair of keys: public and private.

  - Public key is used for encryption, private key for decryption.

  - Slower than symmetric cryptography due to complex mathematical operations.

  - Eliminates the need for secure key exchange.

  - Enables digital signatures and key distribution.

  - Examples include RSA (Rivest-Shamir-Adleman) and ECC (Elliptic Curve Cryptography).

  - Asymmetric: digital signature

Differences between stream ciphers and block ciphers? (=> Confidentiality Protection)

- Stream ciphers encrypt one bit or byte of plaintext at a time, often in real-time, generating a continuous stream of ciphertext.

- Block ciphers encrypt fixed-size blocks of plaintext (e.g., 64 or 128 bits) at a time, padding may be required for uneven blocks.

- Stream ciphers are often faster and more suitable for real-time communication, while block ciphers are more versatile and commonly used for data encryption where data can be processed in blocks.

Usage of asymmetric encryption  (=> Confidentiality Protection)

- Secure key exchange: Used to establish shared secret keys for symmetric encryption.

- Digital signatures: Ensures authenticity and integrity of messages or data.

- Key management: Facilitates secure distribution and management of encryption keys.

- Secure communication channels: Helps establish secure channels for communication over untrusted networks

Usage of Hybrid Protocols  (=> Confidentiality Protection)

- Hybrid protocols combine the strengths of both symmetric and asymmetric encryption to achieve efficiency and security.

- Typically, a hybrid protocol involves using asymmetric encryption for key exchange and symmetric encryption for actual data transmission.

- For example, in SSL/TLS protocols, asymmetric encryption (such as RSA) is used for initial key exchange and authentication, while symmetric encryption (such as AES) is used for bulk data encryption, ensuring both security and performance.

Difference between MACs and digital signatures? (Integrity protection of messages)

- MACs (Message Authentication Codes):

  - Uses a secret key and a message to generate a fixed-size authentication tag.

  - Provides integrity protection and authentication.

  - Typically symmetric-key based.

- Digital Signatures:

  - Generated using the sender's private key and the message.

  - Verifiable using the sender's public key.

  - Provides integrity protection, authentication, and non-repudiation.

  - Typically asymmetric-key based.

## LTE Integrity

| Messages/Signaling Information: | Voice Calls: |
| --- | --- |
| confidentiality + integrity protection | only confidentiality protection |
| block cipher | stream cipher |
| AES in CBC-MAC mode | AES-CTR |
| Messages processed in blocks, each block undergoes several separate transformations | AES-CTR key is used to generate keystream Encryption and decryption happens bit-by-bit: keystream XOR message |
|  | performance ↔ security, block cipher is more secure but not well suited for real-time voice calls |
|  | aLTEr Attack possible due to missing integrity protection |

## Attack: aLTEr Stream Cipher attack

- Attacks Integrity of data

- Change encrypted traffic without knowing plain text or keystream

- Attack is possible due to XOR and missing integrity check

- Can be used for DNS redirection to malicious websites

- Receiver and sender use same keystream to encrypt/decrypt stream

- Attacker can flip/change bits

- Can often only be used to create a gibberish message, but it has been proven that it is possible to manipulate the message properly

## Need to know:

SS7 protocol

Attack: Rogue SS7 Operator (3 parts attacks, 3 steps)

Attack: Stealing Money via SS7 (5 steps)

Location Privacy definition

Location Triangle (3 parts + predictions)

GSM / UMTS Tracking Possibilities (3 parts)

LTE Architecture (Features, encryption + protection)

LTE Connection Setup (7 steps + encryption capabilities)

LTE Impersonation/Billing Fraud (10 steps)

LTE Privacy Tradeoffs (3 parts)

LTE Availability Security Tradeoff

Difference between Symmetric and Asymmetric Cryptography + Example Algorithms/standard

Difference Between Block and Stream Cipher

Usage of asymmetric Encryption + Which part of CIA?

Difference between MAC and digital signature

LTE Integrity: Messages vs Vouce Calls

Attack aLTEr Attack

# 5. Cellular: Attacks on LTE and 5G

## Attack: Eavesdropping Encrypted LTE Calls (ReVoLTE)

LTE voice and media calls use VoLTE service

- bit by bit stream cipher encryption via keystream XOR message

- All stream ciphers are vulnerable to keystream reuse attack!

ReVoLTE Attack:

(1) Target Call: Alice calls Bob, attacker Eve records the call

(2) Keystream Call: Eve calls Alice immediately after the first call

- Theoretically, new keys should be negotiated

- In practice, base stations often use the same keystream

- Flawed implementation due to unclear specification

## 5G Security Issues (in a nutshell)

- Equipment Manufacturers (Huawei) could have modified their hardware to spy on other countries (Smartphones, 5G)

- Huge Attack Surfaces due to interconnected IoT devices and high data rates (cars, smartphones, …)

- 5G authentication doesn’t have explicit threat model and security goals in documentation

  - IMSI Catcher doesn’t work anymore, as SUPI (=5G IMSI) isn’t sent in clear text anymore

  - But: Other tracking and impersonating attacks still possible

## 5G Authentification

Serving Network is a Base Station

SQN = Sequence Number (SQN from Subscriber and Homenetwork should be the same)

UMTS hat auch SQN!

pubK and privK are used to encrypt/decrypt SUPI, by computing SUCI which is the encrypted SUPI

This Authentication makes IMSI catchers useless due to the encryption used

## 5G AKA Protocol

AKA: Authentication and Key Agreement protocol

Das hier ist wie oben, bloß mit resynchronisierung (SQN von Subrsiber und Home Network können out of sync sein)

Grafik müssen wir nicht können, aber wir müssen wissen dass die procedure eigentlich genau so wie ist bei UMTS. Es gibt:

- AUTN (Authentifizierung des Home Networks gegenüber des Subscribers)

- AUTN und Challenge R wird von Home Network über Serving Network zu Subsriber durchreroutet		und serving Network bekommt einen key KSEAF

- Subrsibe rüberprüft on AUTN okay ist

- Subrisber sendet RES (Response) to Serving Network, Sertving Network leitete weiter an Home Network

- Home Network überprüft ob Response passt, wenn ja sendet Home Network SUPI and Serving Network

- Authentifizierung erfolgreich

- Failure 1 - Sync Failure: Authentifikation erfolgreich, außer in der kommunikation ist die SQN out-of sync (? Nachrichten gingen verloren), dann wird Sync failure gesendet, udn ein resync procress gestartet

- Failure 2 - MAC Failure: Authentifikation erfolgreich, außer MAC Failure passiert (wird in RFID nochmal genauer gesprochend)

  - Failures erlauben Attacken, siehe RFID

## Cellular Security & Privacy: 12 Lessons Learned

- No “security by obscurity” in cryptography

- Provide mutual authentication

- Provide security (confidentiality + integrity) in every part of the system

- Crypto algorithms should be easy to change

- Consider future technology developments and adequately powerful attackers in threat analysis

- Provide transparent technology development processes

- Implementing backward compatibility can leave old vulnerabilities exploitable

- Management of pseudonyms should be specified & implemented very carefully

- Management of non-secure modes should be specified & implemented very carefully

- Specifications should warn clearly about possibilities of insecure implementations

- Availability, reliability, performance measures as well as new applications can have unforeseen security & privacy consequences

- Specifications should precisely define security goals and threat model

## Need to know:

Attack: Eavesdropping Encrypted LTE Calls (ReVoLTE)

5G Security Issues (in a nutshell)

5G Authentification

5G AKA Protocol

Cellular Security & Privacy: 12 Lessons Learned

# 5. Wi-Fi: Wi-Fi, OAN, WEP

## Wi-Fi (Wireless Fidelity, IEEE 802.11 since 1997)

8 Goals:

- Global + Seamless Operation

- Low Power (for battery use)

- No special license or permissions needed

- Robust transmission

- Simplified spontaneous cooperation of users

- Easy to use

- Safety (low radiation)

- Security

Different Versions over the years:

- WEP

- WPA

- WPA2

- WPS

- WPA3

## Network Types: Infrastructure and Ad-hoc

## Wi-Fi Network Infrastructure

## Connecting to Wi-Fi Network

### Scanning

- Active: STA sends probes into the medium and waits for an answer

- Passive: STA listens into the medium for beacon signals

- Beacons: signals sent by AP for announcing its presence

### Authentication

- Open system (no cryptographic protocols)

  - STA sends “authenticate request”

  - AP sends “authenticate response”

- Shared key (using WEP/WPA/WPA2/WPA3 cryptographic algorithm)

  - Challenge-response protocol

  - WEP: broken

  - WPA: partially broken

  - WPA2: more or less okay

    - Password guessing attacks, insider attacks: possible “by design”

  - WPA3: public key crypto

### Passive Scanning + Open System Authentication

### Active Scanning + Open System Authentication

### Active Scanning for Available Networks

## Attacks: Open Network Protection -How to prevent alien STAs from joining the WLAN - and how an attacker can join anyway

### Solution 1: Hidden ESSID

How it works:

- APs do not send beacons, wait for STAs to ask for a particular ESSID

- = Security by Obscurity! => Bad

How to attack it: Joining Attack

- Sniff till some STA sends a probe request to the hidden WLAN

- Replay Attack: Send probe request for the same WLAN name

Privacy: STAs actively looking for hidden WLANs at every location

### Solution 2: MAC Address Filtering

How it works:

- APs only answer to the probes or authentication requests of STAs with known MAC addresses

- Security by Obscurity => Bad

How to attack:

- Sniff allowed MAC addresses

- Schange (spoof) your MAC addressed to sniffed allowed one

## Attack: Rogue Access Point (Evil Twin Attack)

Setup the Attack:

- Attacker sets up AP, this AP impersonates the legitimate AP

- Broadcasts beacon signal with the ESSID of the legitimate AP

- => Evil Twin

Attack:

- Device connect to the AP with the strongest signal

- Evil Twin beacon can be made the strongest (by going near the victim’s device)

What can be done with an Evil Twin:

- Send fake login pages to user device, steal passwords etc.

- Forward Internet traffic (=sniff all clear text information, including login info)

- Phishing: Use DNS spoofing to redirec to evil servers (redirect traffic from My Bank to evil fake version of the Bank)

Defense:

- VPN

- SSL/TLS

- but:  no general defense

## WEP (Wired Equivalent Privacy, 1999)

### Goal

Province same security as a wired connection

### How it works

One permanent shared master key for all network members, via RC4 stream cipher

- RC4 used to implement PRNG for generating the keystream

  - PRNG = Pseudo Random Number Generator

  - RC4 is considered insecure nowadays

- IVs (initialization vectors)

  - Produces different keystream for each wireless frame

  - Sent in the clear together with the frame

### Secure?

No, real time traffic decryption and key recovers possible, RC4 is insecure

## Cryptography Basics: Stream Ciphers

- Pseudo random number generator PRNG()

  - Generates bit sequences: 0010111010101…

- Cryptographically secure

- Secret key k = seed

- PRNG(k) = pseudo-random bit stream

- Bitwise XOR with the message

  - Similar to one time pad, but no perfect secrecy

## Cryptography Basics: Pseudo Random Number Generator (PRNG)

- PRNG algorithm should be public (Kerckhoff’s Principle 2)

- Seed is secret (=key)

- Cryptographically secure PRNG:

  - 1) Infeasible to predict next output bit of the output with probability

  - significantly better than ½ without knowing the seed

  - Even if all previous bits of output are known

  - 2) Infeasible to predict previous bits of output if the current state becomes known

No markov chain for you, sorry!

## WEP Encryption and Decryption

### Encrypt:

### Decrypt

## Attack on WEP: - Cryptographic Attacks

Real-Time Key recovery possible

Weaknesses in RC4 and WEP IV management led to more efficient

cryptographic attacks

- Problem/Security Issue: PRNG seed = IV || WEP key

- Key recovery possible after capturing some traffic

## Attack on WEP: Non-crypto Attacks

Exist but were not explained in the lecture.

Attacks that do not cryptographically break RC4

## Need to know:

WiFi Goals (8 parts)

Infrastructure and Ad-hoc Networks

Wi-Fi Architecture (6 parts)

Connecting to Wi-Fi Networks (3 different ways)

Authentication in Wi-Fi (2 different possibilities)

Scanning for Wi-Fi Networks (3 different ways)

Attack: Alien STA joins network

Attack: Evil Twin Attack

WEP (Goal, how it works)

WEP Encryption and Decyrption

Attack on WEP: - Cryptographic Attack

# 6. More Wi-Fi: WEP

## One Time Pad

## Cracking Security if Two Times Pad is used

Encrypt:

- m1 XOR k = c1

- m2 XOR k = c2

If c1 and c2 known  how to find out m1 and m2?

- c1 XOR c2 = m1 XOR m2

m1 XOR m2 is not (pseudo)random! (in contrast to c1 and c2 )

- Can be decrypted via Crib Dragging

Crib Dragging:

- Assume that some common word (e.g., “hello“) appears in m1, starting with position 1

- XOR “hello“ to (m1 XOR m2) starting with position 1

- Result: assumed first 5 characters of plain text of m2

- Try different words for the start of the message

## WEP: Initialization Vector (IV)

- IVs are used to produce different keystream for each wireless frame

- Sent in the clear together with the frame

- IV needs to be public and unique (= public one-time-pad)

- Why is IV needed: Receiver needs additional information that indicates where to start the PRNG

## Attack on WEP: IV Reuse

Serious problems if the same IV is used with the same key more than once => “two-times” pad

C1 XOR C2 = (M1 XOR keystream) XOR (M2 XOR keystream) = M1 XOR M2

## Attacks on WEP: Two other IV-based Vulnerabilities

- PRNG Restart

  - WEP PRNG may be restarted every time a laptop is restarted

  - At restart, IV is set to 0 and incremented with every sent packet

- IV too short

  - WEP IVs are 24 bit long

  - => IVs are reused after around 7 hours (11Mbps sends 700 packets per sec)

  - APs use the same key for months and years

  - Many messages are encrypted with the same keystream

## Attack on WEP: CRC-based Attack

= Attack on Message Integrity

### CRC (Cyclic Redundancy Check):

- a hash function that generates a checksum based on the contents of the data packet or message.

- WEP uses CRC as error-detection code

- CRC is not cryptographically secure, only catches random bit flips, but not bit flips by an intelligent attacker

- CRC is a linear function: CRC(a XOR b) = CRC(a) XOR CRC(b)

### The Attack

Attackers exploit weaknesses in CRC by crafting specially manipulated packets that maintain CRC integrity, allowing them to modify packet contents without detection. Due to this vulnerability, attackers can launch attacks such as packet injection or modification, compromising the integrity of the WEP-encrypted communication.

### Cryptographic Details about the attack

Consider M’ = M XOR d (d can be arbitrarily chosen)

- M not known but construction of a meaningful M’ still possible

  - Example: flip a bit in a message with a payment amount

  - Send M’ with the same IV as M

- The new manipulated message can be calculated via: C XOR (d||CRC(d))

  - Where C is the original message: C = keystream XOR (M || CRC(M)

## Attack on WEP: Replay Attack on Access Control

Shared Key Authentication with/without encryption

Challenge-response: demonstrate possession of the WEP key

## Summary: 4 WEP Design Issues

Key management

- Global master key per ESSID

  - If key leaks, key replacement in all devices is needed

  - No key management protocol for key replacement

  - No session keys, master key directly used

    - Large amount of traffic is encrypted with the same key

    - Combined with other weaknesses, leads to attacks

IV management

- IV size too small (24 bits): reuse

- Real key size small: WEP keys are 54bit and 128 bit small AND!!! include IV

  - real key is 40 bits long and not 64 => Real Time brute force

  - real key is 104 bit long, no brute-force, but cryptographic attack in real time

Cryptography:

- RC4: flawed usage of IVs makes cryptographic attacks possible

- RC4 is by now considered insecure, but WEP weaknesses could be exploited even before the latest RC4 flaws were discovered

- No cryptographic integrity protection (only CRC) Message change possible

Authentication protocol design

- Replay protection is not guaranteed

Implementations

- IV reuse on restart

## WEP: 4 Lessons Learned

- Don’t use master keys directly to encrypt communication

- Integrate key management into the system

- Key distribution and update

- When using cryptographic algorithms, always ask experienced cryptographers how to do this properly:

- WEP uses RC4 in an inappropriate way

- Be extremely careful when using stream ciphers

- Think about reuse of initialization vectors and other components that should be used only once

- Consider replay attacks

- Always use cryptographically secure integrity protection

- Shared secret key: MAC = Message Authentication Code

- Public key crypto: digital signatures

## Need to know:

One Time Pad

Two Times Pad: Why it is insecure + Crib Dragging

WEP: IV

Attack: Three Attacks on WEP by abusing IV (prerequisite and attack)

Attack: CRC-based attack on WEP

Attack: Replay Attack on Access Control of WEP

4 WEP Design Issues

WEP: 4 lessons learned

# 6. More Wi-Fi: WPA and WPA2

## WPA and WPA 2 (Wireless Protected Access)

### Improvements of WPA and WPA2 in comparison to WEP

- 128-bit keys, 48-bit IVs

- Temporal keys (derived from master key)

- Cryp. secure MIC (message integrity check)

- 4-way handshake: Authentication and key management

WPA: uses RC4 (temporal measure till WPA2 came out), worked on WEP hardware

WPA: uses AES in CCM mod for authenticated encyrption, required new hardware

## WPA2 Key Hierarchy

### WPA2-PSK (Pre-Shared Key)

- PMK = PBKDF2(passphrase,salt)

- Salt: ESSID (network name)

- Passphrase: 8 to 63 printable ASCII characters

- PBKDF2: Password-Based Key Derivation Function 2

  - Cryptographically secure function for password generation

### WPA2-Enterprise

Individual PMK for each STA-AP pair and each session

EAP: Extensible Authentication Protocol

- Authentication and key agreement

- Many variations (Asymmetric/hybrid cryptography (TLS), Symmetric cryptography, Password authentication)

- 802.1X Server = Authentication Server

## Four-Way Handshake

## Best practices for Authentication and key management protocols

- Clear security goals and trust assumptions

- Protocol messages should include names of all participants

- Generated cryptographic keys should depend on input by all untrusted participants

- Different keys should be used for encryption and for authentication

  - Exceptions: specifically designed authenticated encryption algorithms, such as AES-CCM used in WPA2 and ZigBee

- Provide replay protection: should not be possible to use message from one protocol instance in another protocol instance

  - Rules for “proper” usage of nonces or timestamps

## Attack: WPA2-PSK Key Cracking

Attack:

- PMK = PBKDF(password, salt), 	where salt is the ESSID (=network name)

- Capture handshake

- Try out passwords using a dictionary

- Rainbow tables precomputed for some most popular network names

- sid, linksys, NETGEAR, default, …

Defense:

- Option 1: SSID: unique, long, sufficiently non-popular…

- Option 2: PSK Passwords

  - Cracking can be done offline + salt is known

  - Depends on computing power

  - Strong passwords make it hard/impossible to crack

  - But: User harassment with strong passwords

- Option 3: Difference to user-selected passwords for online services

  - Offline attacks only possible with database leak

  - Passwords are hashed and salted, salts are unpredictable

  - More defense mechanisms in backend

  - But: User harassment with strong passwords

## Attack: Insider Attacks on WPA2-PSK

- Each insider STA can eavesdrop if it captures ANonce and SNonce of another STA

- Because the same PMK is used for all STAs

## Attack: Insider Attack of WPA2 via Hole 196

### Attack:

Can be used to eavesdrop traffic even in WPA2-Enterprise

### Defense:

- Static ARP tables, or monitoring of ARP tables at the STAs

- Wireless intrusion detection systems (e.g., at APs)

  - Attack can be only detected in the air traffic!

- Change WPA2?

  - Usage of individual keys instead of GTK

  - Usage of digital signatures for authentication of broadcast message

## Attack: Key Reinstallation Attack

Mut zur Lücke, den scheiß merk ich mir nicht. Hier ne ChatGPT zusammenfassung

- Exploits a vulnerability in the WPA2 protocol's 4-way handshake process.

- Attackers can force reinstallation of an already-in-use encryption key.

- Occurs due to improper handling of cryptographic handshake messages.

- Allows attackers to decrypt and intercept data transmitted over the Wi-Fi network.

- Attackers can also inject malicious content into encrypted traffic.

- KRACK does not require knowledge of the Wi-Fi network's passphrase.

- Vulnerable devices include those running vulnerable implementations of WPA2, affecting a wide range of devices.

- Mitigation involves patching affected devices and updating Wi-Fi access points and client devices.

## WPA2 Security Summary

Key hierarchy

- Master key never used for traffic encryption

- Transient keys for each session

- Individual keys for each STA-AP pair

  - WPA2-PSK: insider attacks possible “by design”

  - WPA2-Enterprise: insider eavesdropping is possible through Hole196

Key management

- WPA2-PSK: pre-shared passwords

  - Password should be able to withstand guessing attacks

- WPA2-Enterprise: authentication server

4-way handshake

- “Kracked”, but can be repaired

## Need to know:

WPA and WPA2 Security Improvements in comparison to WEP (4 parts)

Why WPA is insecure/exists but was still proposed/used

WPA2 Key Hierarchy (7 keys)

WPA2-PSK + PMK + Calculation

WPA2-Enterprise + PMK

Four Way Handshake

Best practices for Auth and key management protocols (4 parts)

Attack: WPA2-PSK Key Cracking (attack + Defense)

Attack: Insider Attacks on WPA2-PSK (Hole 196) + Defense

Attack: Key Reinstallation Attack + KRACK

WPA2 Security SUmmary (3 parts)

# 7. More Wi-Fi

## WPA3

- Designed to fix the WPA2-PSK password cracking attacks

- Uses SAE = Simultaneous Authentication of Equals = “Dragonfly”

  - Executed before the 4-way-handshake

  - Uses public key cryptography

  - For each session: generates a new PMK with high entropy out of the Wi-Fi password

- Based on a well-known provably secure cryptographic protocol called “Dragonfly”

## Attack on WPA3: Dragonblood Attacks

- Side channel attacks on SAE which leak information about Wi-Fi password

- Cryptographic Attack

- Password cracking by attempted downgrading to WPA2

  - fMeasures against downgrading are implemented

  - Password cracking still possible

## Defenses against Downgrading

## Lessons Learned: WPA2/3

- Strong cryptography & established cryptographic protocols offer strong protection

- Designing and implementing interactive security protocols is very difficult

  - Specifications should be very clear and unambiguous (state machines, meticulous pseudo code)

  - Krack attack discovered after 14 years

- WPA2-PSK: User-generated passwords will be weak

  - Brute-forcing passwords should (ideally) not be possible from captured handshakes  WPA3

  - Avoid user-generated passwords

- Insider attacks should be considered in the protocol design

- Vulnerabilities in secure protocols can result from unexpected interactions with insecure protocols

  - Hole 196: ARP was never meant to be secure against impersonation

- Design process for cryptographic algorithms should be open

  - WPA3 was designed by the Wi-Fi Alliance without public discussion / comments

- Backwards compatibility usually opens ways for downgrade attacks

  - Downgrade from WPA3 to WPA2 is difficult to prevent in networks that use both

## Need to know:

WPA3 (Why it is more secure)

Attack on WPA3: Dragonblood

WPA3: Defenses against Downgrading

7 lessons learned WPA2/3

# 7. Zigbee

## IEEE 802.15.4

### Design Goals in comparison to Bluetooth and Wi-Fi

Physical (PHY) + Medium Access Control (MAC) layers

- Longer range than Bluetooth

- Lower power, data rate and complexity

- Multi-month to multi-year battery life

- Sensor data, control commands, no voice or multimedia

- Small code size, less operations to implement

## ZigBee Versions/Timeline

- ZigBee 2004

- Zigbee 2006 (Cluster library, encryption support, frame authenticity)

- ZigBee Pro (2007): New security model, software features and enhanced security

- ZigBee 3.0 (current): Discontinuation of Profiles, easier and streamlined communication

## Clusters & Profiles

- Clusters: sets of message types related to a certain device function, like Protocols, Security, …

- Profiles: sets of message types related to a certain application scenario, like Home Automation (discontinued in ZigBee 3.0)

## ZigBee Home Automation (ZHA) Profile

- Interface between devices in a smart home

- Channels don’t overlap with Wi-Fi

- Clusters used: General, Lighting, Security, ….

- discontinued in ZigBee 3.0

## ZigBee Light Link (ZLL) Profile

- For consumer grad lighting devices (smart lamps)

- Similar clusters to ZHA

- Can co-exist with ZHA in theory, in reality it never worked out well due to compatibility issues between different profiles (integrate ZLL smart bulb into a ZHA network)

- discontinued in ZigBee 3.0

## ZigBee PANs (simplified)

- PAN = Personal Area Network

- Each ZigBee deployment consists of at least one PAN, PANs are logically separated (one per household)

- Mesh topology, multiple devices and routing hops,  High reliability achieved through multiple paths

## ZigBee Nodes & Networks

## Security Goals of ZigBee

- Security goals: ZigBee specifications do not define security goals

  - We define our own one: Legitimate user should always keep control over devices and their data

- Confidentiality: Only legitimate entities are allowed to access data and commands sent within the network.

- Integrity: Data and commands sent within the network are not tampered with.

- Availability: The functionalities and data of the devices in the network are

continuously available to all legitimate entities.

- Authenticity: The receiver is able to reject commands and data sent by illegitimate entities.

## Attacker Model

- Can eavesdrop on wireless communication

- Can inject packets in wireless communication

- Cannot access nodes physically

- Cannot access nodes remotely via Internet

## Keys in ZigBee 3.0

### Network key (shared among all devices in a PAN)

- = Each PAN uses a shared symmetric key for communication

- Authenticated encryption: AES-CCM (state-of-the-art)

- Node starting a new network generates the network key for this network

### Link Key (used for commissioning)

- Commissioning: process of starting a new network or joining a new node to an existing network

- Network key is transmitted to newly joining devices using a link key

- Secure commissioning = cornerstone of ZigBee network security

## Network Types and Link Keys

## Touchlink (ZLL) Master Key

- Protected by NDA

- Leaked on Twitter in March 2015

## EZ-Mode Commissioning

Invoked by user action (e.g., pushing button on IoT device)

- IoT device scans for open networks to join (for 3 minutes)

- If network is open, it responds to device with network information

- IoT devices decides wether to joint he network and which link key to use

- Network key encrypted with link key is transferred to the joining IoT device

## Touchlink Commissioning

- Initiator: usually remote control or router

- Target: light bulb or other ZigBee device with dedicated functions

- Both possess the NDA-protected Touchlink link key, aka:

  - ZLL (ZigBee Light Link) Master Key

- Exchange of identifiers (individual per commissioning event)

  - Initiator sends TrID: transaction identifier, 32-bit, randomly generated

  - Target sends RsID: response identifier, 32-bit, randomly generated

- Identify request: initiator asks target to identify itself if many targets are available

  - Light bulb: blink several times

  - Optional operation

- Network join end device request

  - Initiator sends network key (NWK) to target encrypted with the master key

  - TrID and RsID are used to make encrypted message different for each commissioning

## Network Key Encryption

## Inter-PAN frames

- Special type of ZigBee frames to transmit touchlink command

- Transmission neither encrypted nor authenticated

- => can be abused

## Active Device Scan

Not an attack, but a prerequisite for all other attacks

- Touchlink commands are accepted by targets only if previously they received a scan request with the same TrID (transaction ID)

- Scan for touchlink-enabled devices in the wireless range

  - Works even if target is already joined to a network

## Attack: Identify Action Attack

- Doesn’t require knowledge of any cryptographic material

- Trigger identify action (e.g., blinking, beeping, dimming) of target device

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Field to specify duration: 16bit  ≈ 65000 seconds ≈ 18 hours => Bulb blinks until it runs out of battery, blocks other operations of the lamp

- Recovery: manually disconnect from power source

## Need to know:

Design Goals of Zigbee

Clusters, profiles, ZHA and ZLL

ZigBee Pans

ZigBee Nodes & Networks

ZigBee 3.0: Networks Key

ZigBee 3.0: Link Key

ZigBee 3.0: Network Types, used keys, which key is known by nodes in which network type

ZLL Master Key

EZ Mode Commissioning

Touchlink Commisioning

Network Key Encyrption

InterPan Frames (what are they, how can they be abused?)

Active Device Scan

Attack: Identify Action Attack

# 8. More Zigbee

## Attack: Reset to Factory-New Attack

- Doesn’t require knowledge of any cryptographic material

- Reset target to the factory-new state

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Threat scenario: access to restricted area

  - Touchlink-enabled door lock

- Reset to factory-new  door probably unlocks

- Recovery: recommission the affected devices

## Attack: Permanent Disconnect Attack

Two attack possibilities

| Change wireless channel of target: network update request | Join target to garbage network |
| --- | --- |
|  | Network key is encrypted using AES-Encryption in ECB-mode Not AES-CCM (authenticated encryption) as for network communication AES-ECB does not support authentication, only encryption (no integrity protection) Attack: – Send a random 128-bit garbage to the target – Target will decrypt “garbage” and join non-existing network with unknown network key |

Recovery: physical reset

- Osram Lightify: turn on 3 seconds, off 5 seconds repeat five times

- Philips Hue: no physical reset possibility found, possibly no user- driven recovery

- Attacker can recover anytime using the same toolkit as for the attack

Possible threat scenarios: DoS, ransom

## Attack: Hijacking - Attack with knowledge of the leaked global master key

- Active attack: requires interaction

- Join target to attacker‘s network

- Send commands: turn on/off, change color, open/close (e.g., door lock)

- Works even if the device is already joined to another network

## Attack: Network Key Extraction - Attack with knowledge of the leaked global master key

- Passive attack: eavesdropping on touchlink commissioning

- GE and Osram: User has no interface (on smartphone app) to trigger Touchlink

- How long should the attacker wait till user commissions a device?

  - „Motivate“ user to re-commission any device by reset-to-factory-new attack

## ZigBee Proximity Check

Limits range of accepting touchlink commands

If receiving signal strength (RSS) > predefined threshold, then send scan response

## Bug in Proximity Check in Smart Bulbs

Affected Smart Bulbs of Philipps, Osram and GE

## Attack: Using Proximity Chek Bug to factory-reset Devices

- A scan request with TrID=0 for a scan request is invalid

=> rejected if received with scan request

- All other inter-PAN commands, if sent with TrID=0, are accepted by the bulb without proximity check (as result of a programming bug)

- Can reset any bulb to factory new without previous scanning and without proximity check

- If a bulb is reset to factory new, it can be joined to new networks without proximity check

## Conclusion of ZigBee Security Analysis

Touchlink commissioning in insecure by design

- A single touchlink device in the network can expose network key

- Global master key cannot be renewed due to backwards compatibility requirements

Recommendations

- Disable touchlink in all ZigBee 3.0 products

- Use EZ-Mode commissioning with install codes (although it has usability drawbacks)

## Lessons Learned ZigBee (3+3)

- Never use a global master key

- Never use signal strength for proximity verification

- It takes a very looooong time to fix bugs in software of IoT devices

Lessons confirmed

- Always precisely define security goals and attacker model

- Security by obscurity fails (NDA-protected keys)

- Encryption without integrity protection fails (AES-ECB networks key encryption

## Establishing Economic Incentives for Security Patching of IoT Consumer Products

Problem

- Consumers don’t want to pay for security

- Manufacturers are unwilling to provide security

- => Regulation needed to avoid those two problems

Solution: Provide officially regulated “security Label”

- Add options for reliably patching vulnerabilities

- Manufacturers have to define update policy for each product, printed as labels on each product and packaging

  - Madatory “No Security updates” label if Manufacturer doesn’t provide updates at all

Problems:

- Ineffective if flaws can’t be patched, Outsourcing debuggin to consumers (brand image damage), Low User Acceptance (do consumers care about updates)

- Users see smart camera etc as more dangerous as for example smart weather stations, even if both could allow atatckers equal entries into a network and other devices

## Need to Know:

Attack: Reset to Factory-New Attack

Attack: Permanent Disconnect Attack (two ways)

Attack: Hijacking - Attack with knowledge of the leaked global master key

Attack: Network Key Extraction - Attack with knowledge of the leaked global master key

Attack: Using Proximity Chek Bug to factory-reset Devices

Conclusion of ZigBee Security Analysis

Lessons Learned ZigBee (3+3)

# 9. Bluetooth and Device Pairing

## Device Pairing

= Key establishment between two devices

- Devices do not share any common secrets

- “Have never met before”

Security goals: secure channel and CIA

- Authentication of both communication partners

- No MitM (man-in-the-middle) attacks, no impersonation (Evil Twin)

- Integrity + confidentiality of communication

## (Original) Bluetooth Idea

- Wireless Personal Area Network (WPAN)

- Universal radio interface for ad-hoc wireless connectivity

- Short range (10 m), low power consumption

- Voice and data transmission, approx. 1 Mbit/s data rate

- Comparison to Infrared:

  - + Wi-Fi experience can be used

  - + Larger coverage than IR (BT can penetrate walls)

  - + Better bandwidth then IR

  - - limited free frequencies (IR doesn’t need licence)

  - - Shielding of BT more difficult (IR can easily blocked)

  - -More interference with other electrical devices

Later versions of Bluetooth also had a low energy mode, better and more secure pairing/cryptography), better IoT support and functionality

## Bluetooth Piconet (System Architecture)

- Collection of bluetooth devices (4 node types)

  - Connected in an ad hoc fashion

  - One unit acts as master and the others as slaves for the

lifetime of the piconet

  - Master coordinates traffic, slaves have to synchronize

  - Standby: not in the net, may try to join

  - P: Parked - Stay synchronized, listen to traffic

- Frequency-hopping spread spectrum radio technology (800 hops per second)

  - Each piconet has a unique frequency hopping pattern

  - Depending on master’s Bluetooth address (48-bit unique ID)

  - first 16 bits: “non-significant address part”: used for

  - frequency hopping

  - next 32 bits: “significant address part”: used for other

  - Bluetooth algorithms, including security

- Participation in a piconet = synchronization to hopping sequence

  - Each piconet has one master and up to 7 simultaneous slaves

(> 200 could be parked)

## Bluetooth profiles

- Represent default solutions for a certain usage model - Basis for interoperability

- Examples: LAN Access Profile, File Transfer Profile, Cordless Telephony profile

## Bluetooth Scatternet (System Architecture)

- = Linking of multiple co-located piconets through the sharing of common master or slave devices

- Devices can be slave in one piconet and master of another

## Bluetooth Security vs GSM/UMTS/LTE and Wi-Fi

- GSM / UMTS / LTE

  - Predefined and centralized security associations

  - Registered users, SIM cards with preloaded keys

- WiFi

  - Managed security associations (infrastructure mode)

- Bluetooth

  - Ad hoc, spontaneous, unmanaged security associations

  - Device pairing

    - Secure channel between intended devices

    - Key establishment

## Legacy Bluetooth Security

- Symmetric Keys, 128-bit

- Encryption based on SAFER+ algorithm

  - Attacks known, less secure than AES

- Custom Stream cipher with man theoretical attacks

  - Actual cipher strength: 60 bit, even with 128 bit key

## Bluetooth Key Hierarchy

Initialization key (function of PIN)

- Temporal key for link key establishment protocol

- Used when devices meet for the first time, or if link key “forgotten”

- Devices can only store a limited number of link keys

Link key

- Generated by both devices from the initialization key

- When devices meet again, they run a protocol to prove the possession of the link key to each other

Encryption key

- For data encryption, generated from the link key

## Initialization Key Generation and Authentication

## Link Key Generation

## Link Key Update

- Use the same protocol as for the link key generation

- Uses previous link key KAB instead of Kinit

- Helps against attackers that are not present permanently

## Encryption Keys

Individual shared key between master A and slave B. How it works:

- Random number generated by A is sent to B

- A and B compute a Key based on the random number and the Link Key

Broadcast Encryption Key. How it works:

- Key generated by master for each session

- Transmitted to slaves using individual shared key

## Bluetooth 1.0-2.0 Vulnerabilities

- Secrecy of encryption key depends on PIN

- No PIN management

  - PINs can be too short and/or too simple

  - Especially if user-defined or default

  - Many car hands-free sets use 0000 or 1234

- PIN cracking attacks possible

## Attack on older Bluetooth: Passive PIN Cracking

Attack on older Bluetooth: Active PIN Cracking

### Active attack 1

- Initialization key generation: start using any PINX, compute Kinit_X

- Initialization key authentication: receive SRESB from B, perform PIN guessing as above

- SRESB contain the right PIN!

### Active attack 2

- Force A and B to repeat pairing

- Impersonate one if the devices and pretend that the device forgot the link key

- “Forgot key” messages are (necessarily!) sent insecurely

## Seven Bluetooth 1.0-2.0 Vulnerabilities

- Bluetooth devices are often configured to accept connections to arbitrary devices and send their BD_ADDR to them. Suer can be tricked to accept connection

Attacks: user tracking, malware, data stealing

- Weak Cryptography (SAFER+ and E0 weaknesses)

- Encryption IV reuse (2 times pad, pads repeat after 23 hours of uninterrupted connection)

- No integrity check in encryption algorithm (CRC used, same as in WEP)

- No end-to-end encryption (intermediate devices can decrypt traffic)

- Security can be switched off through negotiation between devices

- Several Implementation Vulnerabilities and Attacks (BlueSniff, …)

  - Remote Execution, Man in the middle, etc

## Six Bluetooth Lessons Learned (or confirmed)

- Implement key or PIN management

  - User-defined “secrets” are usually too weak

  - Default “secrets” are not secret

- Use asymmetric crypto for key / PIN management

  - Symmetric crypto is (always?) susceptible to cracking

- Use strong cryptography

  - AES better than SAFER+

- Protect integrity of encrypted communication

  - Don’t use CRC for cryptographically secure integrity guarantees

- Implement properly

  - Good standards should facilitate good implementations

- Do not rely on communication range for security

  - Radio waves propagate beyond the “official” communication range

## SSP: Secure Simple Pairing (starting with Bluetooth v2.1)

More secure connection establishment and key exchange via SSP with Diffie-Helmann:

- Devices initiate pairing process

- Devices Authenticate themselves to each other

- Diffie-Hellman is used to securely generate a Encryption Key for further communication

## Cryptography: Asymmetric Crypto Key Exchange

### Unauthenticated

- Secure against passive eavesdropping

- Vulnerable to man-in-the middle (MitM) and impersonation (Evil Twin) attacks

- Usual realization: Diffie-Hellmann (DH) key exchange

### Authenticated

- Secure against MitM & Evil Twin attacks

- Often requires user involvement

- Enter PIN, compare two strings, scan QR code, …

  - Usual realization: authenticated DH key exchange

  - Other possible protocols (not used in Bluetooth): Dragonfly Key Exchange, SAE (Simultaneous Authentication of Equals)  many variants, one of them used in WPA3

## Diffie-Hellman (DH) Key Exchange

### What is DH?

- DH is an asymmetric cryptography process to generate/exchange a key that can then be used for symmetric encryption

- During DH the channel is open, everyone can listen

### Public Values

- p: large prime number (1024 bits)

  - Multiplicative group modulo p: {1, 2, …, p-1}

- g: 1 < g < p-1

  - g is generator of multiplicative group modulo p

### DH Key Exchange Protocol

## DH Key Exchange Security

Save against passive eavesdrop atatcker

Active attacker:

- Can attack and eavesdrop with Man in the middle attack

## Attack: Man-in-the-Middle Attack on DH

## Authenticated DH (+MANA)

=> Against DH MitM attacks

Human-assisted authentication

MitM still possible via a hash collision on h,, but infeasible if output of hash function is long enough. Output needs to be at least 64 bits long

## Need to know:

Device Paring

(Original) Bluetooth Idea

Piconet

Bluetooth Profiles

Bluetooth Scatternet

Bluetooth Security vs GSM/UMTS/LTE and Wi-Fi

Legacy Bluetooth Security

Bluetooth Key Hierarchy

Initialization Key Generation and Authentication

Link Key Generation

Link Key Update

Encryption Keys

Bluetooth 1.0-2.0 Vulnerabilities

Attack on older Bluetooth: Passive PIN Cracking

Attack on older Bluetooth: Active PIN Cracking ( 2 versions)

Seven Bluetooth 1.0-2.0 Vulnerabilities

Six Bluetooth Lessons Learned

SSP: Secure Simple Pairing (starting with Bluetooth v2.1)

Cryptography: Asymmetric Crypto Key Exchange - Authenticated and Unauthenticated

Diffie-Hellman (DH) Key Exchange (What is DH, Public Values, Exchange Protocol)

DH Key Exchange Security (passive and actzive attacker)

Attack: Man-in-the-Middle Attack on DH

Authenticated DH (+MANA)

# 10. More Bluetooth

## Bluetooth 2.1+ Pairing: Numeric Comparison

Requirement: Both devices must have displays and “yes/no” buttons

Authenticated DH: integrity checking with the 6-digit number as authenticator

Process:

- Both devices must have displays and “yes/no” buttons

- During the pairing procedure, both devices display a 6-digit hash of the public DH keys

- User has to compare the numbers

  - If the numbers are the same, push “yes” on both devices, otherwise push “no”

- This method is provably secure

## Basic Numeric Comparison

(Bluetooth Numeric Comparison is a little bit more complicated and secure, we consider a basic protocol

| A generates a long random value R1 (e.g., 128 bits), computes h1 = h(R1), and sends h to B B generates a long random value R2, sends it to A A: after having received R2, sends R1 to B B checks if h(R1) = h1 – If yes, B computes v2 = f(PKa,PKb,R1,R2), otherwise it aborts A computes v1 = f(PKa,PKb,R1,R2) Both devices display v1 and v2, user accepts if v1=v2 |  |
| --- | --- |

Security:

However: User might fail to compare 6 digit numbers correctly by mistake

## Bluetooth 2.1+ Pairing: Passkey Entry

Requirements: Device A has a display, device B has a keypad, or both device have keypads, but no displays

Authenticated DH: 6-digit shared secret

Process (method 1):

- One device displays a randomly generated secret 6-digit number N

- User enters N into another device

- Then the devices authenticate their DH key using N

Process (method 2):

- User “generates” N and enters it into both devices

- Then the devices authenticate their DH key using N

Security:

- Only secure if N is really random, and it N is a nonce

Attack possibilities:

- Devices sometimes have fixed passwords

- USer generated predictabel passwords (like 00000)

- Attacker can interrupt protocol, user starts another run with the same N

Security Assumptions

- Passkey should be difficult to guess

- Passkey can be used only once

- Last step (user checks whether both devices displayed OK) is necessary

  - User has to confirm to A that B displayed “OK”, and vice versa

- Can we avoid user interaction in this protocol if the device that did not display “OK” stops communication?

  - No, this is not sufficient. Assume that B displayed “FAILED” and stopped, then A can still display “OK” due to the attack from the previous slide.

Bluetooth Passkey Entry

- Recommended but not mandatory procedure

  - Random generation of passkey for each pairing

  - One device generates and displays passkey, user enters it into another device

- Vulnerabilities

  - n rounds for n-bit passkey, passkey reuse not prohibited

  - User-generated passkeys possible: will not be random

  - Passkey reuse attack: DoS on the pairing  user will likely start again with the

  - same passkey

- General problem: Usage of a cryptographic protocol that violates protocol’s security assumptions

## Basic Passkey Authentication Protocol

### Prerequisites and notations

- Devices A and B both have a keypad and a display

- MAC(K,x): message x encrypted under key K using Message Authentication Code

- || concatenation

- A and B exchanged DH public keys PKa and PKb

### Authentification

### Necessity of User Check on Both Devices

Only one party can be impersonated, requires brute forcing, and either no user check or accidental user check (=user isn’t looking)

## Bluetooth 2.1+ Pairing: OOB (out-of-band)

Requirements: Uses NFC or other OOB channel + user interaction

DH executed over Bluetooth, DH public keys authenticated via OOB

## Bluetooth 2.1+ Pairing: Just works

Intended to be used when all other options impossible, e.g., the devices do not have any input capabilities nor OOB

Unauthenticated Diffie-Hellman

## Attack: Degradation to Just Works Attack

Man in the middle attack

Spoof device capabilities: Convince devies that communication partner does not have any input options, even if they have them

## Attack: Method Confusion Attacks

MitM attack on BLE pairing

– One device executes Passkey Entry (PE) with the MitM device

– Another device executes Numeric Comparison (NC) with the MitM device

– Needs a jammer to suppress some messages

– User unable to spot the difference (user study)

Works for Bluetooth Classic with some restrictions

## Unauthenticated DH in BLE

## Bluetooth Pairing: 6 Lessons Learned

- For ad hoc pairing, use public key crypto

- Be careful when providing less secure options

  - Degradation attacks

- Security assumptions of crypto protocols are important

- Do not rely on user-generated secrets

  - They will not be random

- Simplify or avoid user interaction

  - Reduces mistakes and misunderstanding

- Explicitly embed into the protocols which methods are used

  - Distinct values for PE and NC  distinguishable by devices, not by users

  - Example: least significant bit of the passkey for PE = 0, of the hash for NC = 1

## Need to know:

Bluetooth 2.1+ Pairing: Numeric Comparison

Basic Numeric Comparison

Bluetooth 2.1+ Pairing: Passkey Entry

Basic Passkey Authentication Protocol

- Prerequisities

- Authentication

- Necessity of User Check on Both Devices

Bluetooth 2.1+ Pairing: OOB (out-of-band)

Bluetooth 2.1+ Pairing: Just works

Attack: Degradation to Just Works Attack

Attack: Method Confusion Attacks

Unauthenticated DH in BLE (Mut zur Lücke?)

Bluetooth Pairing: 6 Lessons Learned

# 10. Device pairing

## Device Pairing

- Key establishment between two devices

  - Devices do not share any common secrets (“Have never met before”)

- Security goals: secure channel

  - Authentication of both communication partners

    - No MitM (man-in-the-middle) attacks, no impersonation (Evil Twin)

  - Integrity + confidentiality of communication

- Idea: use out-of-band (OOB) channels to exchange authentication information

  - OOB channel is different from the primary wireless channel

  - Devices setup a connection over the primary channel, e.g., using unauthenticated DH

  - Use the information exchanged over the OOB channel to prevent MitM attacks

## Possible OOB Channels

- User interaction (e.g., MANA protocols)

  - Enter PIN on both devices, push buttons, compare strings

- Additional channels (usually also involve user interaction)

  - Physical connection (USB, docking port)

  - Secondary transmission technology (infrared, NFC)

  - Audio (devices beep)

  - Visual (blinking LEDs, scanning of QR codes)

  - Objects (moving objects between devices)

## WPS: WiFi Protected Setup + WPS Options

Goal: Usable and secure pairing of STA and AP

- Push-Button: unauthenticated DH

  - Push a button (sometimes a virtual one in the interface) on both STA and AP at (nearly) the same time

  - Unauthenticated Diffie-Hellmann key exchange is activated

  - No guarantee against malicious (or unintended) connections to wrong devices

- PIN: DH authenticated by shared secret

  - Enter PIN of the AP into the STA interface, or vice versa

- NFC (Near Field Communication)

  - Bring devices near to each other, such that STA can read network configuration from the RFID tag on the AP (optional and usually not implemented)

## Attack: WPS AP-PIN Cracking

AP-PIN authentication:

- Enter 8-digit PIN of the AP into the STA interface

  - PIN is printed on AP  same PIN is used for all STAs

- AP PIN: 8 digits

  - PIN brute forcing requires 10 8 (100 million) attempts

Attack: There is a WPS design flaw (?)

- AP first sends confirmation (or rejection) for first 4 PIN digits

- Then AP sends confirmation (or rejection) for last 4 PIN digits

- Last PIN digit is a checksum for the other 7 digits

- Number of attempts reduced to 11000

- Implementation flaw

  - No back off after unsuccessful attempts or too short back offs

  - Try out PIN=0, if not success, then try out PIN=1  till success

  - Depending on WPS implementations, takes some minutes or some hours to crack the PIN by sending candidate PIN parts to the AP

## Need to know:

Device pairing (what is it, security goals, general idea and security)

Possible OOB Channels

MANA Protocols

WPS: WiFi Protected Setup + WPS Options

Attack: WPS AP-PIN Cracking

# 11: Device pairing in research and OOB channels

## Resurrecting Duckling

Protect gadgets belonging to the same person or organization

RD is an access control policy

Secure Transient Association

– Secure: devices should obey commands of their users, and only of them

– Transient: resold or broken devices can be de-associated

## Network-in-a-box (NiaB)

Goal: Enrollment into a secure WiFi network

Problem: Clients suualyl have to download and configure digitial certificates (to avoid evil twin attack), not practical for average user

Idea of NiaB: Use infrared communication as OOB channel

- Place STA (Laptop) near AP to use infrared OOB comms

- AP has a root certificate (public/private key pair)

- STA generates a public/private key pair

- OOB: Exchange hashes of public key

- In-Band: Run WPA-Enterprise enrollment based on TLS

NiaB Advantages:

- Very usable, fast, no user mistakes

- Secure (no evil twin)

NiaB Disadvantages:

- Line of sight needed

- Difficult to adjust devices is AP is placed in a bad spot (like the ceiling)

## Seeing Is Believing

Device capabilities

- Device A can generate and display a bar code

- Device B can scan the displayed barcode

- Visual OOB channel

Authentication of a public key by integrity checking

- OOB transfer

- User is not required to enter or compare anything

How it works:

- Device A authenticates its public key to device B

  - Displays hash of the key embedded into the barcode

  - Sends the key to B over radio channel

- Device B

  - Scans the barcode, extracts the hash

  - Compares extracted hash with the hash of the previously received key

## Shake Well Before Use

## T2Pair (Touch to Pair)

## IoTCupid

## Object-based Pairing: Wanda Idea

Goal:

- Key establishment over the main wireless channel

- No previous secrets shared by devices

Wand idea:

- “Magic wand” imparts credentials to target devices

- – Wand uses two radio antennas

- – Uses radio technology already available on devices (WiFi, Bluetooth, ZigBee, ...)

- – Requires software changes on target devices, but no hardware changes

- Basic operations

- – Detect: Wand detects that it is in close proximity of the target device

- – Impart: After detection, Wand transmits secrets to the target device

Wand Detect

Wand Impart

Wanda Protocols

•••Common Key

– Impart network credentials on the new device

– Example: add new device to the home WiFi network

– Wand receives WiFi credentials through a USB connection to the AP

– User does not have to remember SSID and password

Unique Key

– Impart a common “fresh” key on two or more devices

Copy & Paste

– Receive information from one device and transfer it to another device

## Lessons Learnt

••••For ad hoc pairing, use public key crypto

Use OOB

Security assumptions of crypto protocol are important

Simplify or avoid user interaction

– Reduces mistakes and misunderstanding

## Need to know:

Resurrecting Duckling

Network-in-a-box (NiaB)

Seeing Is Believing

Shake Well Before Use

T2Pair (Touch to Pair)

IoTCupid

Object-based Pairing: Wanda Idea

Lessons Learnt

# 12. RFID

## RFID (Radio-Frequency Identification)

=> Identification and tracking using radio waves

Architecture

- Integrated circuit (store and process data, modulate/demodulate signal)

- Antenna (receive power, transmit ID, extended functionality possible)

## Passive RFID

- No battery

- With or without extended capabilities (e.g., crypto)

## Battery Assisted Passive (BAP) RFID

- Battery

- Does not actively transmit information

  - Wait until a reader contacts them

  - Use battery for answering (larger communication Rage)

## Active RFID

- Battery

- Microcontroller, radio, memory

- Active communication patterns possible

  - Transmit without being asked by a reader

  - Communication between tags

## Passive RFID with Sensors

- No battery

- Changes in environment set an additional bit in memory

- Applications

  - Monitoring physical parameters

  - Tamper detection

  - Detect radiation or bacteria

## RFID Applications

- Security and Safety (access control, verification, e-documents)

- Tracking (supply chain, hospital)

- Authenticity (medicine)

- Electronic Payment (ApplePay)

## Barcodes

- Optical machine-readable representation of data

  - 1D: Barcode

  - 2D: QR-Code

- Readers (scanners): Cheap and accurate

## Barcodes vs. RFID

## EPC (Electronic Product Code)

Unique identifier for every physician object anywhere in the world for all time

## EPCglobal

- Defines the EPC structure as a freely available standard

- Defines decoding and encoding rules for efficiently storing EPCs on RFID tags and other data storages (like barcodes)

- Generation 1 (deprecated): 0 - Read only, 1 - write once, read many, field programmable

- Generation 2 - Classes:

  - 1: Passive, stores EPC ID, password, kill switch (tag self-destroys after transmitting individual Kill PIN)

  - 2: Passive Extended (memory, authenticated access control)

  - 3: semi passive (battery, sensors)

  - active (battery, tag-to-tag communication, ad hoc networking)

## EPCglobal Tag Format

## EPC: Reading Multiple Tags Issues

Example: Retail - all goods in a pallet or shopping car

Challenges:

- Packaging, Water, Plastic reflect and shield RF signal

- Poorly orientated antennas

- Collisions!!!

## EPC RFID Tag Collision

RFID anti-collision protocols are necessary because multiple RFID tags may be present in the reader's interrogation field simultaneously, and the reader needs to communicate with each tag individually without interference.

## Query Tree Protocol (EPC Generation 1)

= a protocol against tag collision when multiple RFID chips are near the scanner

- Initialization: The reader sends out an initial query command to the RFID tags within its range.

- Response from Tags: Each RFID tag responds with its unique identifier (ID).

- Binary Tree Formation: Based on the received IDs, the reader constructs a binary tree data structure. This tree structure helps in organizing the tags into groups for subsequent interrogation.

- Query Tree Execution: The reader then starts a series of queries based on the binary tree structure. It selectively addresses groups of tags using a binary search algorithm, narrowing down the range of IDs in each query iteration until it identifies the individual tags.

- Tag Identification: Each tag that receives a query responds accordingly, either by acknowledging its presence or by providing additional information as required.

## Query Slot Protocol (EPC Generation 2)

- Initialization: The RFID reader sends out a Query command to all tags within its range.

- Tag Response: Each tag that receives the Query command calculates a random backoff time based on its EPC (Electronic Product Code) identifier. This helps prevent collisions between tags that might respond simultaneously.

- Slotting: After the backoff time elapses, tags that are ready to respond transmit their data in response to the Query command. The reader receives these responses and acknowledges them.

- Tree Formation: Based on the received responses, the reader organizes the tags into groups. This grouping is based on the binary tree structure, similar to the Query Tree Protocol in EPC Gen 1, but with some enhancements for efficiency.

- Query Execution: The reader continues sending Query commands, using the information from the previous responses to refine its search and identify individual tags.

- Tag Identification: Tags respond to the Query commands, providing their IDs or other relevant information. The reader acknowledges these responses and continues the process until all tags are identified

## Query Tree vs. Query Slot

## 4 Security Threats in EPC

- Corporate espionage (read out confidential data about supply chain organization gathered through unauthorized readers)

- Infrastructure (Jamming Attacks: can disrupt RFIDs of an organization)

- Competitive marketing (unauthorized readers can gather and compile statistics in supermarkets, this is highly confidential marketing research data)

- RFID malware

## RFID Malware

- Send invalid ID tag number to scanner

- And then: SQL injection in the RFID data base

- Or: Self replication - inject other RFID tags when sending requests (if the tags are writable), infect other databases

## 7 Privacy Threats in EPC

- Action: Monitor people’s behavior through ther action with tagged objects (check if person tries to steal product from shelf)

- Association: Consumer is associated with the unique tag number of the product

- Location: Secretly track people through their associated tags using secret unauthorized readers

- Preference: Determine consumer preference without asking by scanning their purchases

- Constellation: People are associated with a “cloud” of multiple RFID tags that permanently identifies them (RFID chips in shoes, clothing, …)

- Transaction: Infer transaction between individuals through the movement of their “clouds” or RFID tags

- Breadcrumb: Association between person and ID tag is not broken even if product is discarded (what if the object is then used to commit a crime?)

## Need to know:

RFID

Passive RFID

Battery Assisted Passive (BAP) RFID

Active RFID

Passive RFID with Sensors

RFID Applications

Barcodes

Barcodes vs. RFID

EPC (Electronic Product Code)

EPCglobal

EPCglobal Tag Format

EPC: Reading Multiple Tags Issues

EPC RFID Tag Collision

Query Tree Protocol (EPC Generation 1)

Query Slot Protocol (EPC Generation 2)

Query Tree vs. Query Slot

4 Security Threats in EPC

RFID Malware

7 Privacy Threats in EPC

# 13: More RFID

## Tag Killing (Ensuring Privacy?)

- EPC tags have “Killing PIN”: tag self-destorys, tag is disabled forever

- Disadvantage: After-sales use impossible

- RFID after-sales use: Warranty or product return without receipt, reminders about medicine in cabinet, warning by washing machines that soem garments cannot be washed …

- Problem: Consumer might not check or be able to check if tag was killed (=> Metro Future Store issue)

## Tag Covering (Ensuring Privacy?)

- Cover and uncover if needed

- Enables after-sale use

- Problem: Inconvvenient, tags ban be hidden by manufacturer, makes theft easier

## Blocker Tag (Ensuring Privacy?)

- Embedded in a bag or carried in pocket

- Readers sends signal => Blocker responds to block reading

- Problem:

  - Collisions, send random noise, …

  - Blocking Issue: Reader gets stopped (reader trying to resolve collisions forever)

  - Anti-theft protection difficult

## Selective Blocker Tags

Blocker tag can be selective

- Privacy zones: Only certain ranges of serial numbers are blocked

- Example: block only part of the tree in the query tree protocol

- Zone mobility: Shops move items into privacy zone upon purchase

- Example: Tags with privacy bit: 1 = blocks identifiers, 0 = allow identifiers

- PIN for privacy bit flipping required to prevent theft

## RFID Bill of Right

- 1. The right to know if a product contains an RFID tag.

- The right to have embedded RFID tags removed, deactivated, or destroyed when a product is purchased.

- The right to first-class RFID alternatives. Consumers should not lose other rights (such as the right to return a product or travel on a particular road) if they decide to opt-out of RFID or exercise an RFID tag’s kill feature.

- The right to know what information is stored inside their RFID tags. If this information is incorrect, there must be a means to correct or amend it.

- The right to know when, where, and why an RFID tag is being read

## RFID: Lessons Learned (so far)

- Think before deploying pervasive computing technologies

  - Smallest computing units can have exceptionally big S&P implications

- RFIDs connect physical and electronic world: especially critical

  - Attacks can happen at all parts of the system

  - Analyze interfaces between the parts of the system

- Don’t forget the backend (RFID malware)

- Implementing security & privacy without cryptography is difficult

- Watch out for implementation mistakes

  - “Kill” scandal in Metro Future Store

- Watch out for hidden embedded functionality

  - Hidden RFID tags in Metro Payback cards

## Electronic Documents with RFID or other proximity cards

- Biometrics passport (reading distance 1m): Face Data, Fingerprints

- Security Issues: Cloning, changing, …

- Privacy Issue: Unauthorized reading

## Passport Access Control Protocols

Basic Access Control (BAC):

- Mandatory, symmetric crypto for mutual authentication

- Key derived from machine-readable passport info

- Prevents scanning from third party

Active Authentication / Extended Access Control (E)

- Optional; Mandatory in EU

- Asymmetric Crypto

- Protects biometric info

- Information is digitally signed to prevent forgery

- Reader authenticates to the passport with a digital certificate

## Basic Access Control

## Example: Cloning E-Passport, 2006

Required physician access to the real card, could fool machine but nor human controller

## Example: Cloning E-Passport, 2007

No need of physical access to the real passport, done by predicting BAC key if passport info (date of birth etc) is known

## Example: Cloning and Changing E-Passport, 2008

- No physician access needed, worked with EAC (extended access control)

- Why did this work? Some readers accept self-signed certificates from anybody…

## Tracking via Replay Attack

• Eavesdrop on a legitimate session between a passport and a reader

– Record the encrypted message from the reader that contains the passport's nonce

• To identify a particular passport, replay this message

– If replayed message is rejected because the MAC check failed, then this is not the same

passport, as the MAC key is unique to each passport.

– If the message is rejected because of a failed nonce, then the MAC check succeeded,

and therefore it is the same passport.

• A failed MAC check is reported noticeably sooner than a failed nonce

## E Documents - Lessons learned

• Consider S&P consequences of the usage of a particular (wireless) technology

– Why are e-passports RFID-based?

• Crypto keys should be (pseudo)random

– Non-random cryptographic material can be predicted

• Side-channel attacks can exploit:

– Underspecified answers to commands

– Timing of protocol messages

## Ghost-and-Leech Attacks on Access Control and Payment Systems

## Skim Clone RFID Tags of car key

## Lessons Learned

• Consider S&P consequences of the usage of a particular (wireless) technology

– Why are e-passports RFID-based?

• Crypto keys should be (pseudo)random

– Non-random cryptographic material can be predicted

• Side-channel attacks can exploit:

– Underspecified answers to commands

– Timing of protocol messages

• Protect against relay attacks in challenge-response protocols

• Security through obscurity fails!

## Need to know:

Tag Killing (Ensuring Privacy?)

Tag Covering (Ensuring Privacy?)

Blocker Tag (Ensuring Privacy?)

Selective Blocker Tags

RFID: Lessons Learned (so far)

Electronic Documents with RFID or other proximity cards

Passport Access Control Protocols

Basic Access Control (Passport ⇔ Reader)

Example: Cloning E-Passport, 2006

Example: Cloning E-Passport, 2007

Example: Cloning and Changing E-Passport, 2008

Tracking via Replay Attack

E Documents - Lessons learned

Ghost-and-Leech Attacks on Access Control and Payment Systems

Skim Clone RFID Tags of car key

Lessons Learned

# Social Lectures

“I would suggest looking for the key terms of Mark Weiser's vision, difficulties with them today, and at the current and future developments:”

- Invisible / disappearing computing

- Calm technology

- Messiness of everyday life

- Seamless versus seamful

- Privacy / control

“Es ist möglich, dass dieses Jahr auch Fragen zu Smart City dazu kommen.”

- Cybernetics

  - Cybernetics is a central historical precursor and guiding idea behind

many contemporary phenomena, especially smart cities, but also

other forms of ubiquitous computing

    - Systems thinking

    - Governance as steering

    - Horizontal networking

    - Organicist thinking

      - Fiber optic cables and sensors – “nervous system”

      - Command & control center – “brain”

      - Fusion of city, machine & nature

    - Frictionless, apolitical understanding of process

- Smart City

“Explain the vision behind the term “ubiquitous computing” as it was

discussed by Mark Weise. What is the role of “calm technology”?”

"Ubiquitous computing," as discussed by Mark Weiser, refers to the idea of integrating

computing technology seamlessly into everyday objects and environments, to the point

where they become invisible to the user.

Calm technology refers to technology that remains in the background of users' attention,

providing information or assistance when needed without causing distractions or disruptions.

3 Device Types: Tabs (organizer, diary, …), Pad (“scrap computers), Board (blackboard).

Capabilities: Wireless networking, cooperation, context-aware (location)

# Exam Questions/Tasks

Achtung: Die Antworten sind nicht immer unbedingt optimal. Bitte versuche den Scheiß auch selbst zu beantworten, um sicher zu gehen dass ihr volle Punktzahl in der Klausur bekommt.

## Introduction

### Define the three security goals + authentication (4P)

Confidentiality: 	Protect against unauthorized (reading) access

Integrity: 		Protect from unauthorized (writing) changes
Availability: 		Make data always available on request by an authorized entity

Authentification: 	Distinguish between authorized and unauthorized entities

### Define information privacy

The claim of individuals, groups, or institutions to determine for themselves

when, how, and to what extent information about them is communicated to others.

### Explain the term “Internet of Things” (2P)

Things of our daily live contain computers and are connected via e.g. the Internet or other networks to exchange data and information. Additionally, sensors of the devices deliver information about the physical environment. Actors (actuators) respond to the physical environment.

### What are (six) special characteristics of security and privacy in IoT in comparison to the traditional Internet?

- Devices are pervasive, everywhere and often invisible

- Bodily and territorial privacy are important topic in IoT S&P.

- Different/new quality and quantity of data

- Profiling: habits, emotions (detected and processed via cameras and audio)

- Devices observe and interact with the physician environment

- Unprecedented data collection scale & attack surface

### Explain Greenfield’s principle X (e.g., “be deniable”) and provide an example of a system that satisfies / does not satisfy it.

Be deniable = Opt-out always possible.

This is not guaranteed for the Windows IoT OS and the normal Windows OS, as they always collect telemetry data and other data when the user interacts with the system. It is not possible to opt-out of that data collection, even when some of them can endanger the privacy of the user.

### Consider system X (e.g., GSM). Perform a security and privacy assessment of X.

System Description (Actors, Assets and Data):

- Telecom. Companies

- Customers

- Manufacturers

- State

- Assets&Data: Devices and their content, communication metadata, communication content, billing, infrastructure

S&P Goals:

- Confidentiality, Integrity and Availability of all assets

- Non-repudiation of calls, privacy of subscribers

Other Goals:

- Fast Connection Establishment, calls to devices in different networks (fixed networks or networks of other companies), voice calls, text messages, transparent billing system, online banking

- Relatively Simple Authentication and Attachment of Mobile Stations to Network

Attackers/Threat Models:

- Criminals, terrorists, (foreign) State(s), rival companies

- Threats: Eavesdropping, Billing Fraud, Voice Call/Test Message manipulation, loss/theft of devices, tracking of customers/mobile stations

- Attackers don’t know parts of the security and encryption process (Security by Obscurity)

Trade-Offs of Goals:

- Security vs.State: GSM is designed insecurely so that law enforcement can track and eavesdrop on criminals

- Cheaper and Easier System vs. Security: GSM uses Security by Obscurity via the A3/A8 algorithm, which was cheaper, but less secure, and eventually lead to massive costs to fix security issues due to the possibility of breaking the encryption

- Simplicity vs Security: GSM Connection Establishment and Authentication is simple (less steps between Mobile Station and GSM network), but less secure

### How would you secure such a system X?

- No “security by obscurity” in cryptography

- Provide mutual authentication

- Provide security (confidentiality + integrity) in every part of the system

- Crypto algorithms should be easy to change

- Consider future technology developments and adequately powerful attackers in

- threat analysis

- Provide transparent technology development processes

### Explain why your security measures meet your security and privacy goals for system X.

- No “security by obscurity”: Ensures CIA goals by ensuring security even if it is published hows encryption and security system works:

- Provide mutual authentication: Avoid billing fraud attack to ensure that

- Provide security (confidentiality + integrity) in every part of the system: Avoid compromising CIA and to allow exploits/atztacks

- Crypto algorithms should be easy to change: To avoid extenbsive costs when crypto algorithms get insecure in future developoment

### Explain the IoT design principle “be self-disclosing”. Give an example of an IoT system/device that does not achieve that goal, and briefly explain why. (3P)

Ownership, usage and capabilities must be easy to find out.

Modern cars: It is not specified which data is collected, stored and processed on the servers of the manufacturers and 3rd party services. Sometimes even the manufacturers are not sure which data is stored (capabilities are not easy to find out)

### Explain the vision behind the term “ubiquitous computing” as it was discussed by Mark Weise. What is the role of “calm technology”?

Ubiquitous Computing:

- “technologies that disappear”

- “ubiquitous invisible computing”

- “computers invisible to common awareness”

- “computers informally enhancing every room”

- “computers entering invisibly into people’s lives”

- “machines that fit the human environment instead of forcing humans to enter theirs”

- 3 Device Types: Tabs (organizer, diary, …), Pad (“scrap computers), Active-Badges ( + Board (blackboard) in SMart Cities).

- Capabilities: Wireless networking, cooperation, context-aware (location)

- Calm Technology: Technology recedes intot he background of user’s attention

### Outline three special characteristics of security and privacy in IoT systems in comparison to security and privacy of the traditional Internet (1.5P)

??? Ist damit dass hier gemeint?

Meine Antwort:

Data: Other data types (environmental data), new quality and quantity of data (systematic and easy surveillance)

System Accessibility: IoT devices are “always on”, unprecedented data collection scale & attack surface

Interactions between systems: Influence and observe physician world, devices are connected and invisible

Folie:

## Cellular Communication

### How does GSM authentication work? (8 P)

Include:

- needed system components;

- cryptographic secrets, cryptographic algorithms and other essential information;

- content of exchanged messages

GSM (Global System for Mobile Communications) (2P)

- segmentation of the area into cells (100m-35km big, radio areas overlap)

- Different frequencies used in neighbouring cells

- Handover of the connection to the next cell if mobile user is traveling

Authentification: (2P)

- Components: SIM Card (IMSI; TMSI)

- Preshared key KSIM in Simcard (128bit)

- symmetric algorithm

Authentification Process (4P):

- Phone (MS) sends TMSI and its encryption capabilities to Base Station Controller (BSC)

- BSC asks home location register (HLR) for auth. data of the IMSI (Communication between BSC and HLR are forwarded by the Mobile Service Switching Center (MSC)

- HLR gets KSIM from its data and generates data for challenge response:

  - Choose random number RAND

  - Encrypt RAND with KSIM using A3/A8

  - Output is SRES (A3: Response to challenge) and KC (A8: symmetric key to encrypt data)

- HLR sends RAND, SRES and K_c back to BSC

- BSC sends RAND and used encryption to mobile devise (MS)

- Device calculates challenge with KSIM => SRES1, K_C1

- Devices(MS) sends SRES1 to BSC

- BSC verifies if result from HLR and MS are the same

  - if yes: auth successful

  - if not: terminate auth as unsuccessful

### Which vulnerabilities does GSM have? Which of these vulnerabilities were corrected in UMTS, and how?

Free Call Attack: Backend Eavesdropping: Attack possible to due enencrypted microwav link between BSC and MSC, this was fixed in UMTS

Security by Obscurity: A3/A8 algorithms was cryptographically insecure and hidden, UMTS

Eavesdropping via IMSI Catcher: Insecure Communication Encryption algorithm A5/1, was replaced by A5/3 and A5/4 in UMTS, Mutual AUthentication was added to UMTS as well

Additionally Better Integrity Checks were added in UMTS (which were vulnerable though

Eavesdropping the A5 key in microwave link: Was fixed by encrypting microwave link im UMTS

Sim Card Cloning: GSM allowed sim card cloning due to using the bad A3/A8 algorithm, allowing a cryptographic attack to clone the sim card

### Which possibilities for eavesdropping of GSM communication do you know?

2 ways:

Eavesdropping communication via IMSI-Catcher and decrypting it due to bad encryption algorithms

Eavesdropping the A5 key to decrypt communication

### What is an IMSI-catcher? How does a IMSI-catcher work? (2P)

- An IMSI catcher is a portable fake (!!!) base station

- Mobile stations (phones) always connect to base station with strongest signal

- => bring IMSI catcher close to target mobile station

- It can require the MS/phone to send its real IMSI instead of its TMSI

- Allows tracking and eavesdropping

### Outline how an IMSI-catcher can be used to eavesdrop on a phone call in GSM. (2P)

- IMSI catcher impersonates the BSC for the MS to get data from MS

- Impersonates MS in front of real network

- set encryption capabilities of fake MS to A5/0 (no encryption => eavesdrop)

- if A5/0 is refused, set to A5/1, record traffic, and break A5/1

### Explain why eavesdropping via IMSI-catcher does not work in UMTS.(2P)

In UMTS, the MS checks the integrity of security algorithms, and the encryption algorithms A5/3 and A5/4 are stronger than A5/1 (2P).

### Which attacks do IMSI-catchers enable?

Eavesdropping off Communication and Tracking of Mobile Stations.

### Which possibilities to clone SIM cards do you know? Which attacks does SIM card cloning enable?

Option 1: Extract the key KSIM from the smart card…but SIM Cards are smart cards, and smart cars are tamper proof => Secret hard to extract

Option 2: Cryptographic Attack

- Option 2.1: Get physical access to SIM card, break the crypto algorithms A3/A8

  - Submit many RAND queries,  analyze SRES response

  - If A3/A8 is cryptographically secure against chosen plaintext attacks, this attack should be infeasible

    - A3/A8 is NOT cryptographically secure

- Option 2.2: Over-the-air cloning (OTA): find out KSIM  from communication

  - Same as above, but with a more restricted number of RAND; SRES pairs

  - Eavesdrop on (RAND, SRES) paris, break the crypto algorithms A3/A8

### Outline how the GSM downgrading attack on UMTS works.

• Assumption: MS implements both GSM and UMTS

• IMSI-catcher impersonates MS in UMTS mode to RNC

• RNC sends to MS: “fresh” authentication token AUTN

– IMSI-catcher breaks up the connection, saves AUTN

Now we have a valid AUTN…

### How can users be tracked in cellular communication? Which countermeasures against user tracking exist?

IMSI Catcher: Pinpoint targeted person with precision up to several meters

TMSI: TMSI can be switched off by BS, TMSI is rarely changed

IMEI: BS can ask mobille phone to transmit their IMEI (feature against phone theft)

### Why should location data be protected? What makes it important?

Location data can be used to determine which person is when and where. It allows tracking and prediction about where a person will be or which person is at a location. it is a privacy issue.

### What is SS7? Which attacks are possible using SS7? How do these attacks work?

SS7 is a protocol used for communication between telecom operators

Attacks require buying access to SS7

Rogue SS7 Operator: Buy access to SS7, allows 4 attacks

- Locate & track:

  - Ask HLR for IMSI of phone number

  - Ask HLR which MSC is this IMSI

  - Ask MSC: At which BTS is this IMSI

- Eavesdrop: “Please send authentication and encryption keys for this TMSI”

- Manipulate: “This IMSI wants its calls/SMS forwarded to my network”

- Steal Money from Online Banking

  - Attacker gains control over victim’s online banking account (via phising or malware)

  - Attacker looks up victim’s phone number (Online banking with OTP(one-time-password) via SMS)

  - Sets up SMS redirect

  - Logs into online banking

  - Start translation, use SMS OTP to verify transaction => Money!

### Which attacks on security and privacy we considered for LTE / 5G? How do they work? (in a nutshell)

LTE - Impersonation/Billing Fraud: Impersonate UE by man-in-the-middle attrack to trick Commercial Network to use unencrypted communication, and by capturing and ascending authentication challenge response from the UE that should be impersonated

LTE- aLTEr Attack: Manipulate Stream Cipher Data integrity, allows DNS redirects and manipulation of data

5G - ReVoLTE Attack: Abuse reuse of keystreams in voice calls, capture traffic between two MS (Alice and Bob), then immediately call Alice, use data to figure out keystream and use to decrypt captured call between ALice and bob

### What are most important lessons learned about pervasive security and privacy on the example of cellular communication?

12 Lessons:

- No “security by obscurity” in cryptography

- Provide mutual authentication

- Provide security (confidentiality + integrity) in every part of the system

- Crypto algorithms should be easy to change

- Consider future technology developments and adequately powerful attackers in threat analysis

- Provide transparent technology development processes

- Implementing backward compatibility can leave old vulnerabilities exploitable

- Management of pseudonyms should be specified & implemented very carefully

- Management of non-secure modes should be specified & implemented very carefully

- Specifications should warn clearly about possibilities of insecure implementations

- Availability, reliability, performance measures as well as new applications can have unforeseen security & privacy consequences

- Specifications should precisely define security goals and threat model

## Wi-Fi

### Outline the WPA2 key hierarchy. Describe for which purpose each key is used. (6P)

### Briefly outline the difference in PMK generation between WPA2-PSK and WPA2- Enterprise. (2P)

WPA2-PSK: PMK is based on a passphrase (1P)

WPA2-Enterprise: PMK is generated during the authentication (1P)

### Describe an attack that is possible in WPA2-PSK, but not in WPA2-Enterprise due to the above difference in PMK generation.(2P)

In WPA2-PSK, the PMK is based on

- a password and

- salt (=network name)

This allows a key cracking attack. Capture the handshake, and try out passwords using a dictionary/tables with the most popular network names and passwords

In Enterprise the PMK isn’t based on a password, attack is not possible.

## What are the most important lessons learned about pervasive security and privacy on the example of WEP?

- Don’t use master keys directly to encrypt communication

- Integrate key management into the system

- Key distribution and update

- When using cryptographic algorithms, always ask experienced cryptographers how to do this properly:

- WEP uses RC4 in an inappropriate way

- Be extremely careful when using stream ciphers

- Think about reuse of initialization vectors and other components that should be used only once

- Consider replay attacks

- Always use cryptographically secure integrity protection

- Shared secret key: MAC = Message Authentication Code

- Public key crypto: digital signatures

### How do security measures „hidden network“ and „MAC address filtering“ work in Wi-Fi? Which security goals do they have? How can these security goals be attacked?

Goal: Prevent alien STAs from joining the network

Hidden Network: APs do not send beacon data, and waits for STAs to actively search for AP by asking for a particular ESSID

Attack: Sniff till some STA sends probe request, replay probe request

MAC Address Filtering: AP only answers/accepts probes or authentication requests from STAs with know MAC addresses

Attack: Sniff allowed MAC addresses, change/spoof your MAC address to sniffed allowed MAC address

Both “security” measures are bad because they rely on security by obscurity, and can easily be defeated

### How does Evil Twin attack work in public Wi-Fi networks? Which further attacks does it enable?

Setup the Attack:

- Attacker sets up AP, this AP impersonates the legitimate AP

- Broadcasts beacon signal with the ESSID of the legitimate AP

- => Evil Twin

Attack:

- Device connect to the AP with the strongest signal

- Evil Twin beacon can be made the strongest (by going near the victim’s device)

What can be done with an Evil Twin:

- Send fake login pages to user device, steal passwords etc.

- Forward Internet traffic (=sniff all clear text information, including login info)

- Phishing: Use DNS spoofing to redirect to evil servers (redirect traffic from My Bank to evil fake version of the Bank)

### Why can Wi-Fi traffic be eavesdropped on much easier than traffic in cellular networks?

Weak Passwords for Wi-Fi Networks allow to crack encryption between STAs and APs, and ot eavesdrop their communication

Wi-Fi relies on passwords, while cellular communication relies on SIM Cards with unique keys and identifiers

### For which purpose are IV (initialization vectors) used by WEP?

It is used as an offset for the PRNG Function (Pseudo-Random Number Generator) to let both AP and STA use the right offset for the keystream generation to encrypt and decrypt traffic

### Which attacks does IV reuse enable? How can IV reuse happen in WEP?

IV Reuse:  IV is reused (two times pad), allows to brute force and decrypt communication between AP and STA

PRNG Restart: WEP PRNG may be restarted every time a laptop it restarted, at restart IV is set to 0, and incremented with every sent packet

IV too short: IVs are reused after a few hours

### Explain using WEP as example why data encryption is not sufficient for achieving data integrity.

ey management

- Global master key per ESSID

  - If key leaks, key replacement in all devices is needed

  - No key management protocol for key replacement

  - No session keys, master key directly used

    - Large amount of traffic is encrypted with the same key

    - Combined with other weaknesses, leads to attacks

IV management

- IV size too small (24 bits): reuse

- Real key size small: WEP keys are 54bit and 128 bit small AND!!! include IV

  - real key is 40 bits long and not 64 => Real Time brute force

  - real key is 104 bit long, no brute-force, but cryptographic attack in real time

Cryptography:

- RC4: flawed usage of IVs makes cryptographic attacks possible

- RC4 is by now considered insecure, but WEP weaknesses could be exploited even before the latest RC4 flaws were discovered

- No cryptographic integrity protection (only CRC) Message change possible

Authentication protocol design

- Replay protection is not guaranteed

Implementations

- IV reuse on restart

### Which design weaknesses does WEP have? What can be learned from them?

- Don’t use master keys directly to encrypt communication

- Integrate key management into the system

- Key distribution and update

- When using cryptographic algorithms, always ask experienced cryptographers how to do this properly:

- WEP uses RC4 in an inappropriate way

- Be extremely careful when using stream ciphers

- Think about reuse of initialization vectors and other components that should be used only once

- Consider replay attacks

- Always use cryptographically secure integrity protection

- Shared secret key: MAC = Message Authentication Code

### What are security goals of WPA2?

### Present key hierarchy of WPA2, explain fro which purposes are all keys used.

### How does 4-way handshake“ work?

### What are the differences in security guarantees between WPA2-PSK and WPA2-Enterprise?

WPA2-Enterprise: Individual PMK for each STA-AP pair and each session

WPA2-PSK: Only one PMK

WPA2-Enterprise: Authentication and key agreement via Authentication Server, lower chance of impersonating devices

### How does WPA2 key cracking work?

Attack:

- PMK = PBKDF2(password, salt), 	where salt is the ESSID (=network name)

- Capture handshake

- Try out passwords using a dictionary

- Rainbow tables precomputed for some most popular network names

- sid, linksys, NETGEAR, default, …

### Which insider attacks are possible in WPA2-PSK and in WPA2-Enterprise? Which countermeasures should be taken against them?

Insider Attack of WPA2 via Hole 196: Works on both WPA2 variants - Defense: Static ARP Tables, Wireless Intrusion Detection System at ARPs, Use individual keys or digital signatures instead of GTK

### Explain the principle behind the WPA2 Key Reinstallation Attack (KRACK) attack.

- Exploits a vulnerability in the WPA2 protocol's 4-way handshake process.

- Attackers can force reinstallation of an already-in-use encryption key.

- Occurs due to improper handling of cryptographic handshake messages.

- Allows attackers to decrypt and intercept data transmitted over the Wi-Fi network.

- Attackers can also inject malicious content into encrypted traffic.

- KRACK does not require knowledge of the Wi-Fi network's passphrase.

- Vulnerable devices include those running vulnerable implementations of WPA2, affecting a wide range of devices.

- Mitigation involves patching affected devices and updating Wi-Fi access points and client devices.

### What are differences in security design between WPA2 and WPA3? Against which attacks does WPA3 protect, compared to WPA2?

WPA3 does this, WPA2 doesn’t do this:

- Designed to fix the WPA2-PSK password cracking attacks

- Uses SAE = Simultaneous Authentication of Equals = “Dragonfly”

- Executed before the 4-way-handshake

- Uses public key cryptography

- For each session: generates a new PMK with high entropy out of the Wi-Fi password

### Explain the principle behind the Dragonblood attack.

- Side channel attacks on SAE which leak information about Wi-Fi password

- Cryptographic Attack

- Password cracking by attempted downgrading to WPA2

  - fMeasures against downgrading are implemented

  - Password cracking still possible

## Key Exchange and Pairing

### Outline how unauthenticated Diffie-Hellman key exchange between two parties A and B works. You can use a drawing, a text, or a combination thereof for this task (8P)

Include in your explanation:

• public system parameters;

• values chosen by A and B;

• calculations done by A and B;

• content of exchanged messages.

Public Values

- p: large prime number				(0.5P)

- g: generator, 1<g<p-1				(0.5P)

### (7P)

### Explain the difference in security properties between unauthenticated and authenticated Diffie-Hellman key exchange. (2P)

Unauthenticated:

- Secure against passive eavesdropping (0.5P)

- Not secure against man-in-the-middle attack and evil twin attack (0.5P)

Authenticated:

- secure against passive eavesdropping, man-in-the-middle and evil twin (0.5P)

- often requires user involvement (like QR code scanning) (0.5P)

### Outline how authenticated Diffie-Hellman key exchange is used for device pairing in Bluetooth Numeric Comparison from the user perspective. Provision of cryptographic protocols is not needed here. (4P)

Include in your explanation:

• Minimum capabilities of devices;

• Values that are being authenticated;

• Actions that the user performs.

Answer:

- Both devices need a display and “yes” and “no” buttons (1P)

- During paring, both devices exchange diffie hellmann public keys (1P)

- Both devices do integrity checking with 6 digit number (1P)

- Both devices show the 6-digit code, user compares the codes, and has to click yes or no on both devices depending on if the code is matching  (1P)

### What are security goals of device pairing?

Security goals: secure channel and CIA

- Authentication of both communication partners

- No MitM (man-in-the-middle) attacks, no impersonation (Evil Twin)

- Integrity + confidentiality of communication

### What are OOB channels used for?

They are used as a second communication channel or action between devices, in order to authenticate DH public keys while DH is executed over the main communication channel (bluetooth).

### Which OOB channels do you know? Provide examples of device pairing methods that use OOB channel X.

- User Interaction (Manual Authentication Protocols)

  - Enter PIN on both devices, push buttons

- Additional Channels

  - Physical connection (USB, docking port)

  - Secondary transmission technology (infrared, NFC)

  - Audio (devices beep)

  - Visual (blinking LEDs, scanning of QR codes)

  - Objects (moving objects between devices)

### How does method Y for device pairing works? Which security guarantees does it provide?

MANA Protocol: Compare PINs of two dervices, press yes/no

Additional Channels:

- Resurrecting Duckling

- Network-in-a-box (NiaB)

- Seeing is Believing

- Shake Well Before Use

  - -

- T2Pair (Touch to Pair)

  - Touch: Movement with random pauses

  - Secure if Attacker can not replicate/mimic movement

- IoTCupid

  - Devices sense different events and compute a group key out of them

  - Is Secure if Attacker is not present in the perimeter

- Wanda Idea: Object-based Pairing

  - Uses (two) Antennas

  - – Detect: Wand detects that it is in close proximity of the target device

  - – Impart: After detection, Wand transmits secrets to the target device

### What are the root causes of security vulnerabilities in WPS?

Attack: There is a WPS design flaw (?)

- AP first sends confirmation (or rejection) for first 4 PIN digits

- Then AP sends confirmation (or rejection) for last 3 PIN digits

- Last PIN digit is a checksum for the other 7 digits

- Number of attempts reduced to 11000

- Implementation flaw

  - No back off after unsuccessful attempts or too short back offs

  - Try out PIN=0, if not success, then try out PIN=1  till success

  - Depending on WPS implementations, takes some minutes or some hours to crack the PIN by sending candidate PIN parts to the AP

## Bluetooth

### What were the goals of developing Bluetooth technology?

- Wireless Personal Area Network (WPAN)

- Universal radio interface for ad-hoc wireless connectivity

- Short range (10 m), low power consumption

- Voice and data transmission, approx. 1 Mbit/s data rate

### What does Bluetooth system architecture look like? (2 Architectures)

| Piconet: - Collection of Bluetooth devices - Master Node (one per Piconet): Coordinates traffic - Slaves (7 slaves can exits): Synchronize with Master - Parked (200 can be parked): Sty Synchronized, listen to traffic - Standby: non in the piconet, might join - Each piconet has unique frequency hopping pattern - Participation in a piconet = synchronization to hopping sequence => Slaves | Scatternet: - = Linking of multiple co-located piconets through the sharing of common master or slave devices - Devices can be slave in one piconet and master of another |
| --- | --- |

### What does device pairing mean? Which security goals does it have?

= Key establishment between two devices

- Devices do not share any common secrets

- “Have never met before”

Security goals: secure channel and CIA

- Authentication of both communication partners

- No MitM (man-in-the-middle) attacks, no impersonation (Evil Twin)

- Integrity + confidentiality of communication

### How does device pairing work in Bluetooth Versions 1.0-2.0?

General Info:

- Symmetric Keys, 128-bit

- Encryption based on SAFER+ algorithm

- Custom Stream Cipher (60 bit strength)

### Explain key hierarchy in Bluetooth 1.0-2.0

Initialization key (function of PIN)

- Temporal key for link key establishment protocol

- Used when devices meet for the first time, or if link key “forgotten”

- Devices can only store a limited number of link keys

Link key

- Generated by both devices from the initialization key

- When devices meet again, they run a protocol to prove the possession of the link key to each other

Encryption key

- For data encryption, generated from the link key

### Explain the three Blueooth 1.0-2.0 key generation protocols.

Initialization key Generation and Authentication:

Link Key Generation:

Link Key Update

- Use the same protocol as for the link key generation

- Uses previous link key KAB instead of Kinit

- Helps against attackers that are not present permanently

Encryption Keys

Individual shared key between master A and slave B. How it works:

- Random number generated by A is sent to B

- A and B compute a Key based on the random number and the Link Key

Broadcast Encryption Key. How it works:

- Key generated by master for each session

- Transmitted to slaves using individual shared key

### Which 8 vulnerabilities does Bluetooth 1.0-2.0 security have?

- PIN Cracking Attacks are possible

- Bluetooth devices are often configured to accept connections to arbitrary devices and send their BD_ADDR to them. User can be tricked to accept connection

  - Attacks: user tracking, malware, data stealing

- Weak Cryptography (SAFER+ and E0 weaknesses)

- Encryption IV reuse (2 times pad, pads repeat after 23 hours of uninterrupted connection)

- No integrity check in encryption algorithm (CRC used, same as in WEP)

- No end-to-end encryption (intermediate devices can decrypt traffic)

- Security can be switched off through negotiation between devices

- Several Implementation Vulnerabilities and Attacks (BlueSniff, …)

  - Remote Execution, Man in the middle, etc

### Explain Bluetooth 1.0-2.0 Passive PIN cracking.

### Explain Bluetooth 1.0-2.0 the two Active PIN cracking attacks.

Active attack 1

- Initialization key generation: start using any PINX, compute Kinit_X

- Initialization key authentication: receive SRESB from B, perform PIN guessing as above

- SRESB contain the right PIN!

Active attack 2

- Force A and B to repeat pairing

- Impersonate one if the devices and pretend that the device forgot the link key

- “Forgot key” messages are (necessarily!) sent insecurely

### Explain SSP: Secure Simple Pairing (starting with Bluetooth v2.1)

More secure connection establishment and key exchange via SSP with Diffie-Helmann:

- Devices initiate pairing process

- Devices Authenticate themselves to each other

- Diffie-Hellman is used to securely generate a Encryption Key for further communication

### What are advantages and disadvantages of symmetric crypto for Bluetooth pairing compared to asymmetric crypto?

Unauthenticated

- Secure against passive eavesdropping

- Vulnerable to man-in-the middle (MitM) and impersonation (Evil Twin) attacks

- Usual realization: Diffie-Hellmann (DH) key exchange

Authenticated

- Secure against MitM & Evil Twin attacks

- Often requires user involvement

- Enter PIN, compare two strings, scan QR code, …

  - Usual realization: authenticated DH key exchange

### How does unauthenticated DH work?

Public Values

- p: large prime number (1024 bits)

  - Multiplicative group modulo p: {1, 2, …, p-1}

- g: 1 < g < p-1

  - g is generator of multiplicative group modulo p

DH Key Exchange Protocol

### Explain the difference in attacker models between unauthenticated and authenticated DH.

Unauth. DF:

- Protects against passive eavesdropping

- Active Eavesdropping still possible

- Man in the middle attack possible

Auth DH:

- Human-assisted authentication

- Protects against active and passive eavesdropping and man in the middle attack

### Which values are authenticated during the authentication phase of MANA protocols?

Numeric Comparison:

Authenticated DH: integrity checking with the 6-digit number as authenticator

Passkey Entry:

Authenticated DH: 6-digit shared secret

OOB:

DH executed over Bluetooth, DH public keys authenticated via OOB (

Just works

Unauthenticated Diffie-Hellman

### How does SSP method X work from the user perspective? On which assumptions does it rely for security? Which user interaction is required?

Numeric Comparison

- User has two devices with displays and yes/no buttons

- User wants to the two devices

- During pairing, both devices show 6 digit hash of the public DH keys

- User has to compare the numbers

  - If the numbers are the same, push “yes” on both devices, otherwise push “no”

- Assumption: User actually compares the numbers probably, and presses the buttons accordingly

Passkey Entry

- Device A has a display, device B has a keypad, or both device have keypads, but no displays

Process (method 1):

- One device displays a randomly generated secret 6-digit number N

- User enters N into another device

- Then the devices authenticate their DH key using N

Process (method 2):

- User “generates” N and enters it into both devices

- Then the devices authenticate their DH key using N

- Assumption:

  - Only secure if N is really random, and it N is a nonce

  - Passkey should be difficult to guess

  - Passkey can be used only once

  - Last step (user checks whether both devices displayed OK) is necessary

OOB (out-of-band)

- Different User Actions possible

  - Use PINs to enter in both devices, use RFID and hold devices close together, use infrared and hold devices closer together

  - Example: “Seeing is Believing”

    - Device A can generate and display a bar code

    - Device B can scan the displayed barcode

    - => Visual OOB channel

- Assumptions:

  - OOB channel is different from the primary wireless channel

  - Devices setup a connection over the primary channel, e.g., using unauthenticated DH

## Zig-Bee

### Outline how Zigbee 3.0 Touchlink Commissioning uses a global master key for joining new devices to an existing network. You can use a drawing, a text, or a combination thereof for this task. (4P)

Include in your explanation:

• participating devices and their roles;

• cryptographic secrets, cryptographic algorithms and other essential information;

• content of exchanged messages.

Answer:

- Initiator: usually remote control or router

- Target: light bulb or other ZigBee device with dedicated functions

- Both possess the ZLL (ZigBee Light Link) Master Key

  - Initiator sends TrID: transaction identifier, 32-bit, randomly generated

  - Target sends RsID: response identifier, 32-bit, randomly generated

- Identify request: initiator asks target to identify itself if many targets are available

- Network join end device request

  - Initiator sends network key (NWK) to target encrypted with the master key

  - TrID and RsID are used to make encrypted message different for each different for each commissioning

### Which two attacks were made possible due to the leakage of the ZigBee Touchlink Commissioning master key? (2P)

Hijack Attack: (0.5P)

- Join target to attacker‘s network (0.25P)

- Send commands: turn on/off, change color, open/close (e.g., door lock) (0.25)

Network key extraction: (0.5P)

- eavesdrop until user joins a new device to the network (or force user to do so by reset-to-factory-new-atatck) (0.5P)

### Outline security architectures of centralized and distributed ZigBee 3.0 networks and compare their security.

### What are link keys in ZigBee 3.0 networks are used for? Which link keys are installed on each ZigBee 3.0 node? Which of them are required, and which are optional?

Q: What are link keys in ZigBee 3.0 networks are used for?

A: Linkey keys are using for commissioning (=starting a new network or joining a new node to the network). It is also used to distribute the network key to newly joining devices

Q: Which link keys are installed on each ZigBee 3.0 node? Which of them are required, and which are optional?

### What are usability advantages of Touchlink commissioning compared to EZ-Mode with install codes?

???

### What is the ZLL master key used for?

The ZLL master key was protected by an NDA, but got leaked on twitter.

It is an (optiona) global key used by distributed ZigBee networks, and can be used for EZ-Mode and Touchlink (although each ahs a different ZLL Master Key).

### Which attacks on Touchlink do not require the knowledge of the ZLL master key?

Attack: Identify Action Attack

- Doesn’t require knowledge of any cryptographic material

- Trigger identify action (e.g., blinking, beeping, dimming) of target device

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Field to specify duration: 16bit  ≈ 65000 seconds ≈ 18 hours => Bulb blinks until it runs out of battery, blocks other operations of the lamp

- Recovery: manually disconnect from power source

Attack: Reset to Factory-New Attack

- Doesn’t require knowledge of any cryptographic material

- Reset target to the factory-new state

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Threat scenario: access to restricted area

  - Touchlink-enabled door lock

- Reset to factory-new  door probably unlocks

- Recovery: recommission the affected devices

### Which attacks on Touchlink require the knowledge of the ZLL master key?

Attack: Hijacking - Attack with knowledge of the leaked global master key

- Active attack: requires interaction

- Join target to attacker‘s network

- Send commands: turn on/off, change color, open/close (e.g., door lock)

- Works even if the device is already joined to another network

Attack Network Key Extraction - Attack with knowledge of the leaked global master key

- Passive attack: eavesdropping on touchlink commissioning

- GE and Osram: User has no interface (on smartphone app) to trigger Touchlink

- How long should the attacker wait till user commissions a device?

  - „Motivate“ user to re-commission any device by reset-to-factory-new attack

### How does <insert attack here> work?

Prerequisite of ALL attacks: Active Device Scan

- Touchlink commands are accepted by targets only if previously they received a scan request with the same TrID (transaction ID)

- Scan for touchlink-enabled devices in the wireless range

  - Works even if target is already joined to a network

Attack: Identify Action Attack

- Doesn’t require knowledge of any cryptographic material

- Trigger identify action (e.g., blinking, beeping, dimming) of target device

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Field to specify duration: 16bit  ≈ 65000 seconds ≈ 18 hours => Bulb blinks until it runs out of battery, blocks other operations of the lamp

- Recovery: manually disconnect from power source

Attack: Reset to Factory-New Attack

- Doesn’t require knowledge of any cryptographic material

- Reset target to the factory-new state

  - Even if the device is already in a network

  - No authentication (inter-PAN frame)

- Threat scenario: access to restricted area

  - Touchlink-enabled door lock

- Reset to factory-new  door probably unlocks

- Recovery: recommission the affected devices

Attack: Permanent Disconnect Attack

Two attack possibilities

| Change wireless channel of target: network update request | Join target to garbage network |
| --- | --- |
|  | Network key is encrypted using AES-Encryption in ECB-mode Not AES-CCM (authenticated encryption) as for network communication AES-ECB does not support authentication, only encryption (no integrity protection) Attack: – Send a random 128-bit garbage to the target – Target will decrypt “garbage” and join non-existing network with unknown network key |

Recovery: physical reset

- Osram Lightify: turn on 3 seconds, off 5 seconds repeat five times

- Philips Hue: no physical reset possibility found, possibly no user- driven recovery

- Attacker can recover anytime using the same toolkit as for the attack

Possible threat scenarios: DoS, ransom

Attack: Hijacking - Attack with knowledge of the leaked global master key

- Active attack: requires interaction

- Join target to attacker‘s network

- Send commands: turn on/off, change color, open/close (e.g., door lock)

- Works even if the device is already joined to another network

Attack: Network Key Extraction - Attack with knowledge of the leaked global master key

- Passive attack: eavesdropping on touchlink commissioning

- GE and Osram: User has no interface (on smartphone app) to trigger Touchlink

- How long should the attacker wait till user commissions a device?

  - „Motivate“ user to re-commission any device by reset-to-factory-new attack

Reset to Factory Attack

See next question

### How is proximity check used in ZigBee 3.0 Touchlink? To which attacks is it susceptible?

ZigBee Proximity Check

Limits range of accepting touchlink commands

If receiving signal strength (RSS) > predefined threshold, then send scan response

Bug:

- A scan request with TrID=0 for a scan request is invalid

=> rejected if received with scan request

- All other inter-PAN commands, if sent with TrID=0, are accepted by the bulb without proximity check (as result of a programming bug)

Reset to Factory Attack:

- Can reset any bulb to factory new without previous scanning and without proximity check

- If a bulb is reset to factory new, it can be joined to new networks without proximity check

### What are root causes of insecure commissioning modes in ZigBee 3.0?

Touchlink commissioning in insecure by design

- A single touchlink device in the network can expose network key

- Touchlink commands are accepted by targets only if previously they received a scan request with the same TrID (transaction ID) => Attacker sends scan requests, Scan for touchlink-enabled devices in the wireless range

  - Works even if target is already joined to a network

- Global master key cannot be renewed due to backwards compatibility requirements

## RFID

### Briefly outline how RFID is used in supply chain management. (2P)

RFID is used to keep track of objects without having to scan them manually (1P)

An RFID scanner is used to scan for objects along the supply chain, each objects has its own (unique EPC) ID code, and the scanner can then send the IDs and other data to a database used for tracking (1P)

### Give one example of a privacy threat that arises in supply chain management due to RFID usage. Justify your answer. (2P)

Spying on people/competitors (1P)

Use unauthorized RFID readers (0.5P) to gather data and information about supply chain management (0.5P)

### Outline privacy implications of electronic documents using e-passports as example. Which two privacy attacks did we consider in the lecture, how do they work at a conceptual level? (without going into the details of messages) (4P)

- Fingerprinting Passport Nationality (1P)

  - Passports of different countries answer differently to various commands from the readers

  - Possible to determine nationality of the owner (1P)

- Tracking via Replay Attack (1P)

  - Eavesdrop on a legitimate session between a passport and a reader

  - Record the encrypted message from the reader that contains the passport's nonce

  - To identify a particular passport, replay this message (1P)

### What are RFID tags, which types and standards do you know, which applications are possible?

RFID (Radio-Frequency Identification) tags are small tags with integrated circuits with an antenna which can transceive power, and they store, send, process and receive data.

Three types:

Passive (no battery),

BAP (Battery Powered Passive): Only responds if it receives incoming data, longer range than passive due to battery

Active Battery Powered: Active communication patterns without “outside stimuli” from a reader

Passive RFID with sensors: Changes in environment set and additional bit in memory

Applications: STAP - Security&Safety, Tracking, Authentication, Payment

### How does Supply Chain Management using EPC (Electronic Product Code) works?

### What are advantages and disadvantages of EPC compared to barcodes?

EPC is more expensive (bardcodes are printed and basically “free”)

EPC works without the need of a line of sight, proper alignment of the reader and product.

EPC has several potential privacy issues, that can make it possible to track and identify people who own/carry products with RFID EPC tags.

…

### Explain protocol X for reading EPC tags.

Query Tree

Query Slot

### Which security and privacy threats exist in EPC systems? Which technical and sociotechnical countermeasures (e.g., RFID Bill of Rights) do you know?

### Which security and privacy aspects should be considered when designing RFID-based documents?

### Explain how BAC protocol works. Explain how attack X on electronic passport documents works.

Passport Access Control Protocols

Basic Access Control (BAC):

● Mandatory, symmetric crypto for mutual authentication

● Key derived from machine-readable passport info

● Prevents scanning from third party

Attacks:

- Cloning

- Tracking via Replay

- Ghost-and-Leech

### Which attacks on RFID-based access control systems do you know? How should countermeasures be implemented?

CLoning E-Passport: Predict BAC key by using passport info like date of birth. Countermeasure: Use Extended Access Control and Certificates.

Tracking via Replay Attack: Eavesdrio encrypted message, replay it, and measure the time it takes to receive a rejection (Side-Channel Attack): If the rejection comes faster, it got rejected because of a wrong MAC, if it takes longer, then it failed due to the nonce

Ghost-And-Leech: Impersonate Reader and Card

Countermeasure: Distance Bounding Protocols to make “ghost” impossible

### Why do tags in Query Slot Protocol don’t send their ID directly?

If two or more tags are in the same slot, then all tags in that slot send their random 16-bit number to the reader => collision. Then the reader sends a number back (which is either a mixture of both/all RN16 numbers or only one of the RN16 numbers), so all or multiple tags get the wrong RN16 number back from the reader.

The tags then notice that the reader got the wrong number back from the reader, and tell that to the reader.

The reader then chooses a new Q=Q+1, sends it to the tags, and all tags that haven’t been inventoried yet adjust/choose new slots.

Random numbers have a lower chance of colliding with each other, which is not the case with using the IDs directly.

It also has a privacy advantage if a random number is used: In Query Tree, the reader also sends the ID of a tag, and usually a higher transmission range than a tag. This gives an attacker two chances to eavesdrop an ID. With query tree, only the tag sends the ID, with a smaller transmission range than a reader.

# Need to know

Pervasive Computing (5 parts)

IoT Enablers (4 parts)

IoT (5 parts)

WAN (+3/4 Examples)

WLAN (+ 1 Examples)

WPAN (+ 3 Examples)

Security (Definition)

Security Goals (CIA) + Authentication

4 Parts of evaluating attackers

Privacy (3 definitions)

S&P in IoT (3 parts)

5 steps of S&P Assessment

5 Design Principles of pervasive systems (A. Greenfield)

GSM (+4 of its features/properties)

GSM cellular network

GSM Architecture:

* NSS with OSS

* HLR

* EIR

* AuC

* MSC

* VLR

* GMSC

* RSS

* MS

* BTS

* BSC

Handover Decision

Call setup

GSM S&P Assesement (4 parts)

GSM Threats, Victims (4 examples) and Attackers

4 GSM Security features

2 GSM Security Risks

9 steps of GSM athentification and voice/SMS encryption

* Grafik!

* IMEI

* IMSI

* TMSI

* BSC

* MSC

* HLR

* Encryption Algorithms

Free Call Attack: Ross Anderson's Hack (7 steps)

Free Call Attack: SIM Card Cloning (2 Options)

A3/A8 Algorithms (secure?)

Kerckhoff's Six Principles (name the mos timportant principle)

Security by Obscurity (+how to do it right)

AES

Attack: Stolen or Lost MS and Countermearues

Attack: Backend Eavesdropping

IMSI Catcher + Countermeasure

UMTS

UMTS Architecture

MS

USIM

NodeB

RNC

MSC

SGSN

VLR

HLR

EIUR

AUC

SS7

UMTS Authentication and Key Agreement (14 steps)

Man-in-the-middle GSM-UMTS Degradation Attack (7 steps)

Why does GSM-UMTS Degradation Attack work?

Problem of backward compatibility

Attack: Eavesdropping Encrypted LTE Calls (ReVoLTE)

5G Security Issues (in a nutshell)

5G Authentification

5G AKA Protocol

Cellular Security & Privacy: 12 Lessons Learned

One Time Pad

Two Times Pad: Why it is insecure + Crib Dragging

WEP: IV

Attack: Three Attacks on WEP by abusing IV (prerequisite and attack)

Attack: CRC-based attack on WEP

Attack: Replay Attack on Access Control of WEP

4 WEP Design Issues

WEP: 4 lessons learned

WPA3 (Why it is more secure)

Attack on WPA3: Dragonblood

WPA3: Defenses against Downgrading

7 lessons learned WPA2/3

Attack: Reset to Factory-New Attack

Attack: Permanent Disconnect Attack (two ways)

Attack: Hijacking - Attack with knowledge of the leaked global master key

Attack: Network Key Extraction - Attack with knowledge of the leaked global master key

Attack: Using Proximity Chek Bug to factory-reset Devices

Conclusion of ZigBee Security Analysis

Lessons Learned ZigBee (3+3)

Bluetooth 2.1+ Pairing: Numeric Comparison

Basic Numeric Comparison

Bluetooth 2.1+ Pairing: Passkey Entry

Basic Passkey Authentication Protocol

Prerequisities

Authentication

Necessity of User Check on Both Devices

Bluetooth 2.1+ Pairing: OOB (out-of-band)

Bluetooth 2.1+ Pairing: Just works

Attack: Degradation to Just Works Attack

Attack: Method Confusion Attacks

Unauthenticated DH in BLE (Mut zur Lücke?)

Bluetooth Pairing: 6 Lessons Learned

Resurrecting Duckling

Network-in-a-box (NiaB)

Seeing Is Believing

Shake Well Before Use

T2Pair (Touch to Pair)

IoTCupid

Object-based Pairing: Wanda Idea

Lessons Learnt

Tag Killing (Ensuring Privacy?)

Tag Covering (Ensuring Privacy?)

Blocker Tag (Ensuring Privacy?)

Selective Blocker Tags

RFID: Lessons Learned (so far)

Electronic Documents with RFID or other proximity cards

Passport Access Control Protocols

Basic Access Control (Passport ⇔ Reader)

Example: Cloning E-Passport, 2006

Example: Cloning E-Passport, 2007

Example: Cloning and Changing E-Passport, 2008

Tracking via Replay Attack

E Documents - Lessons learned

Ghost-and-Leech Attacks on Access Control and Payment Systems

Skim Clone RFID Tags of car key

Lessons Learned